Troubleshooting Exam

- Which of the following is not an example of negative expansion?

- Cracked Burst Disk

- Leaking Expansion Valve

- Bowl/ Jacket Fill Valve

- Test Jacket Drain Valve

- To insure the system is stable, it should be able to hold on zero for:

- 30 seconds

- 5 minutes

- 20 minutes

- 2 minutes

- The maximum test time for high pressure cylinder testing is 30 seconds at test pressure. – F

- Each day of testing, you need to verify system accuracy using the:

- Master Gauge

- Calibration Cylinder

- Eddy Current Machine

- 50 Gram Weights

- Per DOT, the retester must show calibration within what amount of cylinder test pressure to be tested that day?

- 100 psi

- 1% of test pressure

- 500 psi

- 1000 psi

- If a retester needs to repeat the test due to a leak, the test should be repeated:

- Once

- Twice

- Three times

- As many times as needed to get a proper test.

- If the water in the bowl is continually rising during the test, you should:

- Let the system finish the test

- Abort the test, bleed the pressure and locate the problem

- Shut off the power

- Tell your supervisor and go for a break

- If the test system experiences negative expansion and aborts occasionally then you might have problems with:

A  The High Pressure Valve

B.  The Head Boot.

C.  Temperature.

D.  A family member.

- All cylinders must be hydrostatically tested. – F

- A plus on a cylinder means it is tested at 10% above standard service pressure. – F

- The hammer test should be performed on all cylinders. – F

- The use of a pressure recorder is optional. – T

- You can plus and star a cylinder if it already has one from a previous test. – F

- Your system should be verified for accuracy every day cylinders are tested. – T

- If any cylinder exceeds its REE, it must be scrapped. – F

- Your calibrated cylinder must be re-calibrated when you notice a drift in the expansion ratings. T

- Elastic Expansion is:

- Inversely proportional to wall thickness

- Proportional to wall thickness.

- The letters CR-MO stand for:

- Chromium Molybdenum Cylinder

- Chrome-Moly Alloy.

- Elastic expansion is the difference between total and permanent expansion.  – T

- If you are having trouble with your test machine and cannot figure out the problem, you should:

- Take a vacation

- Contact your local cable company

- Contact Galiso at: 1-800-854-3789

- Cry