AZ Pump Instructions

PUMP NOT RUNNING PROPERLY: Check to be sure air pressure is reaching the pump, check to be sure air filter is not clogged; clean or replace as required. Check to be sure fluid filter is not clogged; clean or replace as required. If the actions do not remedy operations, a general maintenance repair may be required

A REPAIR KIT PART NUMBER 38-11-3084 IS AVAILABLE AND CONTAINS ALL THE PARTS NORMALLY REPLACED DURING A STANDARD MAINTENANCE REPAIR.

GENERAL NOTE: Before installation of new parts, all o’ rings, seals, and bumpers should be lubricated with silicone grease, Vaseline or other suitable lubricant.

Air Motor Assembly

TO REPLACE THE PACKING IN THE HYDRAULIC CYLINDER: It is not necessary to dismantle the Air Motor. Proceed as follows:

Hydraulic cylinder

Disconnect the air supply line. 
Unscrew the Air Motor from the hydraulic cylinder, the Hydraulic Piston, Will be removed with the Air Motor.

Seals will be located here Kit P/N 40-11-3033

The packing, P/N 40-11-3033 consisting of 1 each o- ring and 2 each Teflon backup Rings, can new be removed and new parts installed.    
When replacing the Air Motor, install a new gasket, if necessary. 
Tighten the Air Motor securely to proper position.

Air Motor assembly

Seal Kit P/N 40-11-3033

TO DISASSEMBLE THE AIR MOTOR: refer to this Parts List, proceed as follows:

Disconnect the air supply line. 
Remove the eight (8) 5/16” bolts around the outside diameter.

To remove the hydraulic piston (P-1-1-140), from the Air Piston,(P-1-48) remove the retaining ring.

The pilot valve assembly (P-1-60), can now be pushed out through the bottom of the Air Piston. Remove the retaining ring (RRT-262) from the head assembly and remove the bearing Assembly (P-1-47), by lifting or prying it out with a hammer handle or similar tool. (The Bearing Assembly has a molded rubber seat on one side, if the rubber is damaged, a new bearing assembly should be installed). 
The valve, (P-l-3), can now be removed. Remove the spring, (P-1-5). The head casting, (P-1-1), has a rubber bumper, (P-1-57), inserted in the upper portion of the body. This bumper acts as a seal and cushion for the valve and should be replaced if worn or damaged.

NOTE: when worn or damaged, the valve, (P-1-3), and the Sleeve, (P-1-2), in which the valve operates can be replaced with a new Valve Assembly, (P-1-111). If installing a new valve assembly, install a new set of o-rings, (28775-144), 4 are required and are included when a replacement assembly is ordered. CAUTION: DO NOT REMOVE the sleeve   (P-l-2), unless a replacement valve assembly is being installed. The valve and sleeve are ground and honed to a very close tolerance and the Sleeve may be damaged when removed from the head casting.

TO REPAIR THE PILOT VALVE ASSEMBLY: (P-1-60), Remove the retaining ring, (RRT-93). Using a spanner wrench or needle nose pliers, remove the seat, (P-1-A-24). The air check assembly, (P-1-A-32), and the spring, (P-1-16), will then drop out and may be inspected for wear or damage. Replace worn or damaged parts and reassemble

TO REPAIR THE HYDRAULIC CHECK VALVES: refer to the Parts List

INLET VALVE ASSEMBLY (P-1-29): remove the internal parts, inspect and replace required. Remove the o-ring, (6227-890 Shore), inspect the ball seat n the check valve body (P-1-21). If worn or damaged a new body should be used. Install a new o-ring, (6227-890 Shore), replace the internal parts and secure the assembly by installing on the hydraulic cylinder, (P-1-11-140)

OUTLET VALVE ASSEMBLY (P-1-130): remove the internal parts, inspect and replace required. Remove the o-ring, (6227-890 Shore), inspect the ball seat n the check valve body (P-1-22). If worn or damaged a new body should be used. Install a new o-ring, (6227-890 Shore), replace the internal parts and secure the assembly by installing the retaining ring, (RRT-50).

TO REASSEMBLE THE PUMP: it is VERY IMPORTANT when reassembling the Air Motor Assembly to be sure that the Air Cylinder, (P-1-4), is in its proper position against the flanges on the pump head, (P-1-1), and that the air cylinder end. (P-1-7).  BEFORE tightening the bolts that clamp together the Air Motor Assembly. Use a soft hammer to position the flanges tightly against the air cylinder, (P-1-4). Bolts should be secured LIGHTLY at first, and then drawn up in sequence until uniform torque has been applied to all eight bolts. (10-12 foot lbs.)