| Col. 1 | Col. 2 | Col. 3 |  | Col. 4 | Col. 5 | Col. 6 | Col. 7 | Col. 8 | Col. 9 | Col. 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UN Number | Shipping Name and Description | Class |  | Packing Group/ Risk Group | Special Provisions | Explosive Limit and Limited Quantity Index | ERAP Index | Passenger Carrying Ship Index | Passenger Carrying Road Vehicle or Passenger Carrying Railway Vehicle Index | Marine Pollutant |
| UN1970 | KRYPTON, REFRIGERATED LIQUID | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 50 |  |
| UN1971 | METHANE, COMPRESSED; or NATURAL GAS, COMPRESSED with high methane content | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN1972 | METHANE, REFRIGERATED LIQUID; or NATURAL GAS, REFRIGERATED LIQUID with high methane content | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN1973 | CHLORODIFLUOROMETHANE AND CHLOROPENTAFLUOROETHANE MIXTURE with fixed boiling point, with approximately 49 per cent chlorodifluoromethane; or REFRIGERANT GAS R 502 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1974 | CHLORODIFLUOROBROMOMETHANE; or REFRIGERANT GAS R 12B1 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1975 | NITRIC OXIDE AND DINITROGEN TETROXIDE MIXTURE; or NITRIC OXIDE AND NITROGEN DIOXIDE MIXTURE | 2.3 (5.1) (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN1976 | OCTAFLUOROCYCLOBUTANE; or REFRIGERANT GAS RC 318 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1977 | NITROGEN, REFRIGERATED LIQUID | 2.2 |  |  |  | 0.125 SOR/2002-306 |  | Forbidden | 50 |  |
| UN1978 | PROPANE | 2.1 |  |  | 29, 42 | 0.125 SOR/2002-306 | 3 000 | 65 | Forbidden |  |
| UN1979 | RARE GASES MIXTURE, COMPRESSED | 2.2 |  |  | SOR/2002-306 | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1980 | RARE GASES AND OXYGEN MIXTURE, COMPRESSED | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1981 | RARE GASES AND NITROGEN MIXTURE, COMPRESSED | 2.2 |  |  | SOR/2002-306 | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1982 | REFRIGERANT GAS R 14, COMPRESSED; or TETRAFLUOROMETHANE, COMPRESSED | 2.2 |  |  | 38 | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1983 | 1-CHLORO-2,2,2-TRIFLUOROETHANE; or REFRIGERANT GAS R 133a | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1984 | REFRIGERANT GAS R 23; or TRIFLUOROMETHANE | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. | 3 (6.1) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN1987 | ALCOHOLS, N.O.S. | 3 |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN1988 | ALDEHYDES, FLAMMABLE, TOXIC, N.O.S. | 3 (6.1) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN1989 | ALDEHYDES, N.O.S. | 3 |  | I | 16 | 0 |  | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN1990 | BENZALDEHYDE | 9 |  | III |  | 5 |  |  | 100 |  |
| UN1991 | CHLOROPRENE, STABILIZED | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. | 3 (6.1) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN1993 | FLAMMABLE LIQUID, N.O.S. | 3 |  | I | 16 | 0 |  | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN1994 | IRON PENTACARBONYL | 6.1 (3) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN1999 | TARS, LIQUID, including road asphalt and oils, bitumen and cut backs | 3 |  | II |  | 5 |  |  | 5 | • |
|  |  |  |  | III |  | 5 |  |  | 60 | • |
| UN2000 | CELLULOID in block, rods, rolls, sheets, tubes, etc., except scrap | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2001 | COBALT NAPHTHENATES, POWDER | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2002 | CELLULOID, SCRAP | 4.2 |  | III |  | 0 |  | Forbidden | Forbidden |  |
| UN2003 | METAL ALKYLS, WATER-REACTIVE, N.O.S.; or METAL ARYLS, WATER-REACTIVE, N.O.S. | 4.2 (4.3) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN2004 | MAGNESIUM DIAMIDE | 4.2 |  | II |  | 0 |  |  | 15 |  |
| UN2005 | MAGNESIUM DIPHENYL | 4.2 |  | I |  | 0 | 1 000 |  | Forbidden |  |
| UN2006 | PLASTICS, NITROCELLULOSE-BASED, SELF-HEATING, N.O.S. | 4.2 |  | III | 16, 38 | 0 |  |  | Forbidden | • |
| UN2008 | ZIRCONIUM POWDER, DRY | 4.2 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
|  |  |  |  | II |  | 0 |  | Forbidden | 15 |  |
|  |  |  |  | III |  | 0 |  | Forbidden | 25 |  |
| UN2009 | ZIRCONIUM, DRY, finished sheets, strip or coiled wire | 4.2 |  | III |  | 0 |  | Forbidden | 25 |  |
| UN2010 | MAGNESIUM HYDRIDE | 4.3 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2011 | MAGNESIUM PHOSPHIDE | 4.3 (6.1) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2012 | POTASSIUM PHOSPHIDE | 4.3 (6.1) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2013 | STRONTIUM PHOSPHIDE | 4.3 (6.1) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2014 | HYDROGEN PEROXIDE, AQUEOUS SOLUTION with not less than 20 per cent but not more than 60 per cent hydrogen peroxide (stabilized as necessary) | 5.1 (8) |  | II |  | 0.5 |  | Forbidden | 1 |  |
| UN2015 | HYDROGEN PEROXIDE, AQUEOUS SOLUTION, STABILIZED with more than 60 per cent hydrogen peroxide; or HYDROGEN PEROXIDE, STABILIZED | 5.1 (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2016 | AMMUNITION, TOXIC, NON-EXPLOSIVE without burster or expelling charge, non-fuzed | 6.1 |  | II |  | 0 |  | Forbidden | Forbidden |  |
| UN2017 | AMMUNITION, TEAR-PRODUCING, NON-EXPLOSIVE without burster or expelling charge, non-fuzed | 6.1 (8) |  | II |  | 0 |  | Forbidden | Forbidden |  |
| UN2018 | CHLOROANILINES, SOLID | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2019 | CHLOROANILINES, LIQUID | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2020 | CHLOROPHENOLS, SOLID | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2021 | CHLOROPHENOLS, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2022 | CRESYLIC ACID | 6.1 (8) |  | II |  | 0.1 |  |  | 1 |  |
| UN2023 | EPICHLOROHYDRIN | 6.1 (3) |  | II | 43 | 0.1 | 1 000 |  | 5 | P |
| UN2024 | MERCURY COMPOUND, LIQUID, N.O.S., excluding mercurous chloride and cinnabar | 6.1 |  | I | 23 | 0 | 1 000 |  | 1 | PP |
|  |  |  |  | II |  | 0.1 |  |  | 5 | PP |
|  |  |  |  | III |  | 5 |  |  | 60 | PP |
| UN2025 | MERCURY COMPOUND, SOLID, N.O.S., excluding mercurous chloride and cinnabar | 6.1 |  | I |  | 0 | 1 000 |  | 5 | PP |
|  |  |  |  | II |  | 0.5 |  |  | 25 | PP |
|  |  |  |  | III |  | 5 |  |  | 100 | PP |
| UN2026 | PHENYLMERCURIC COMPOUND, N.O.S. | 6.1 |  | I |  | 0 | 1 000 |  | 5 | PP |
|  |  |  |  | II |  | 0.5 |  |  | 25 | PP |
|  |  |  |  | III |  | 5 |  |  | 100 | PP |
| UN2027 | SODIUM ARSENITE, SOLID | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2028 | BOMBS, SMOKE, NON-EXPLOSIVE with corrosive liquid, without initiating device | 8 |  | II |  | 1 |  | Forbidden | Forbidden |  |
| UN2029 | HYDRAZINE, ANHYDROUS | 8 (3) (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2030 | HYDRAZINE, AQUEOUS SOLUTION with more than 37 per cent but not more than 64 per cent hydrazine, by mass; or HYDRAZINE HYDRATE with more than 37 per cent but not more than 64 per cent hydrazine, by mass | 8 (6.1) |  | II |  | 0.5 |  | Forbidden | Forbidden |  |
| UN2031 | NITRIC ACID, other than red fuming, with more than 70 per cent nitric acid | 8 (5.1) |  | I |  | 0 | 3 000 | Forbidden | Forbidden |  |
| UN2031 | NITRIC ACID, other than red fuming, with not more than 70 per cent nitric acid | 8 |  | II |  | 0.5 |  | Forbidden | Forbidden |  |
| UN2032 | NITRIC ACID, RED FUMING | 8 (5.1) (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2033 | POTASSIUM MONOXIDE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2034 | HYDROGEN AND METHANE MIXTURE, COMPRESSED | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN2035 | REFRIGERANT GAS R 143a; or 1,1,1-TRIFLUOROETHANE | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | Forbidden |  |
| UN2036 | XENON, COMPRESSED | 2.2 |  |  | SOR/2002-306 | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN2037 | GAS CARTRIDGES without a release device, non-refillable; or RECEPTACLES, SMALL, CONTAINING GAS without a release device, non-refillable | 2.1 |  |  |  | 0.125 SOR/2002-306 |  |  | 1 |  |
| UN2037 | GAS CARTRIDGES without a release device, non-refillable; or RECEPTACLES, SMALL, CONTAINING GAS without a release device, non-refillable | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 1 |  |
| UN2038 | DINITROTOLUENES, LIQUID | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2038 | DINITROTOLUENES, SOLID | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2044 | 2,2-DIMETHYLPROPANE | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN2045 | ISOBUTYLALDEHYDE; or ISOBUTYRALDEHYDE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2046 | CYMENES | 3 |  | III |  | 5 |  |  | 60 | PP |
| UN2047 | DICHLOROPROPENES | 3 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2048 | DICYCLOPENTADIENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2049 | DIETHYLBENZENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2050 | DIISOBUTYLENE, ISOMERIC COMPOUNDS | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2051 | 2-DIMETHYLAMINOETHANOL | 8 |  | II |  | 0.5 |  |  | 1 |  |
|  |  | (3) |  |  |  |  |  |  |  |  |
| UN2052 | DIPENTENE | 3 |  | III |  | 5 |  |  | 60 | P |
| UN2053 | METHYL ISOBUTYL CARBINOL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2054 | MORPHOLINE | 8 (3) |  | I |  | 0 | 3 000 |  | 0.5 |  |
| UN2055 | STYRENE MONOMER, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2056 | TETRAHYDROFURAN | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2057 | TRIPROPYLENE | 3 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2058 | VALERALDEHYDE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2059 | NITROCELLULOSE SOLUTION, FLAMMABLE with not more than 12.6 per cent nitrogen, by dry mass, and not more than 55 per cent nitrocellulose | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
|  |  |  |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2067 | AMMONIUM NITRATE FERTILIZERS: uniform non-segregating mixtures of ammonium nitrate with added matter which is inorganic and chemically inert towards ammonium nitrate, with not less than 90 per cent ammonium nitrate and not more than 0.2 per cent combustible material (including organic material calculated as carbon) or with more than 70 per cent but less than 90 per cent ammonium nitrate and not more than 0.4 per cent total combustible material | 5.1 |  | III | 37, 68 | 5 |  |  | 25 |  |
| UN2068 | AMMONIUM NITRATE FERTILIZERS: uniform non-segregating mixtures of ammonium nitrate with calcium carbonate and/or dolomite, with more than 80 per cent but less than 90 per cent ammonium nitrate and not more than 0.4 per cent total combustible material | 5.1 |  | III | 37, 68 | 5 |  |  | 25 |  |
| UN2069 | AMMONIUM NITRATE FERTILIZERS: uniform non-segregating mixtures of ammonium nitrate/ammonium sulphate, with more than 45 per cent but not more than 70 per cent ammonium nitrate and not more than 0.4 per cent total combustible material | 5.1 |  | III | 37, 68 | 5 |  |  | 25 |  |
| UN2070 | AMMONIUM NITRATE FERTILIZERS: uniform non-segregating mixtures of nitrogen/phosphate or nitrogen/potash types or complete fertilizers of nitrogen/ phosphate/potash type, with more than 70 per cent but less than 90 per cent ammonium nitrate and not more than 0.4 per cent total combustible material | 5.1 |  | III | 37, 68 | 5 |  |  | 25 |  |
| UN2071 | AMMONIUM NITRATE FERTILIZERS: uniform non-segregating mixtures of nitrogen/phosphate or nitrogen/potash types or complete fertilizers of nitrogen/ phosphate/potash type, with not more than 70 per cent ammonium nitrate and not more than 0.4 per cent total added combustible material or with not more than 45 per cent ammonium nitrate with unrestricted combustible material | 9 |  | III | 37 | 5 |  |  | 200 |  |
| UN2072 | AMMONIUM NITRATE FERTILIZER, N.O.S. | 5.1 |  | I | 37, 68 | 0 | 1 000 |  | 1 |  |
|  |  |  |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 25 |  |
| UN2073 | AMMONIA SOLUTION, relative density less than 0.880 at 15 °C in water, with more than 35 per cent but not more than 50 per cent ammonia | 2.2 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN2074 | ACRYLAMIDE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2075 | CHLORAL, ANHYDROUS, STABILIZED | 6.1 |  | II |  | 0.1 |  | Forbidden | 5 |  |
| UN2076 | CRESOLS, LIQUID | 6.1 (8) |  | II |  | 0.1 |  |  | 5 |  |
| UN2076 | CRESOLS, SOLID | 6.1 (8) |  | II |  | 0.5 |  |  | 25 |  |
| UN2077 | alpha-NAPHTHYLAMINE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2078 | TOLUENE DIISOCYANATE | 6.1 |  | II | 43 | 0.1 | 1 000 |  | 5 |  |
| UN2079 | DIETHYLENETRIAMINE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2186 | HYDROGEN CHLORIDE, REFRIGERATED LIQUID | 2.3 (8) |  |  |  | 0 | 25 | Forbidden | Forbidden |  |
| UN2187 | CARBON DIOXIDE, REFRIGERATED LIQUID | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 50 |  |
| UN2188 | ARSINE | 2.3 (2.1) |  |  | 38 | 0 | 0 | Forbidden | Forbidden |  |
| UN2189 | DICHLOROSILANE | 2.3 (2.1) (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2190 | OXYGEN DIFLUORIDE, COMPRESSED | 2.3 (5.1) (8) |  |  | 38 | 0 | 0 | Forbidden | Forbidden |  |
| UN2191 | SULFURYL FLUORIDE; or SULPHURYL FLUORIDE | 2.3 |  |  |  | 0 | 25 | Forbidden | Forbidden |  |
| UN2192 | GERMANE | 2.3 (2.1) |  |  | 38 | 0 | 0 | Forbidden | Forbidden |  |
| UN2193 | HEXAFLUOROETHANE, COMPRESSED; or REFRIGERANT GAS R 116, COMPRESSED | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN2194 | SELENIUM HEXAFLUORIDE | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2195 | TELLURIUM HEXAFLUORIDE | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2196 | TUNGSTEN HEXAFLUORIDE | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2197 | HYDROGEN IODIDE, ANHYDROUS | 2.3 (8) |  |  | 38 | 0 | 500 | Forbidden | Forbidden |  |
| UN2198 | PHOSPHORUS PENTAFLUORIDE, COMPRESSED | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2199 | PHOSPHINE | 2.3 (2.1) |  |  | 38 | 0 | 0 | Forbidden | Forbidden |  |
| UN2200 | PROPADIENE, STABILIZED | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | Forbidden |  |
| UN2201 | NITROUS OXIDE, REFRIGERATED LIQUID | 2.2 (5.1) |  |  |  | 0 | 3 000 |  | Forbidden |  |
| UN2202 | HYDROGEN SELENIDE, ANHYDROUS | 2.3 (2.1) |  |  | 38 | 0 | 0 | Forbidden | Forbidden |  |
| UN2203 | SILANE, COMPRESSED | 2.1 |  |  | 38 | 0.125 SOR/2002-306 | 25 | Forbidden | Forbidden |  |
| UN2204 | CARBONYL SULFIDE; or CARBONYL SULPHIDE | 2.3 (2.1) |  |  | 38 | 0 | 500 | Forbidden | Forbidden |  |
| UN2205 | ADIPONITRILE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2206 | ISOCYANATE SOLUTION, TOXIC, N.O.S.; or ISOCYANATES, TOXIC, N.O.S. | 6.1 |  | II | 16, 23 | 0.1 | 3 000 | Forbidden | 5 | • |
|  |  |  |  | III | 16 | 5 |  | Forbidden | 60 | • |
| UN2208 | CALCIUM HYPOCHLORITE MIXTURE, DRY with more than 10 per cent but not more than 39 per cent available chlorine | 5.1 |  | III |  | 5 |  | Forbidden | 25 |  |
| UN2209 | FORMALDEHYDE SOLUTION with not less than 25 per cent formaldehyde | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2210 | MANEB; or MANEB PREPARATION with not less than 60 per cent maneb | 4.2 |  | III | 45 | 0 |  |  | 25 | P |
|  |  | (4.3) |  |  |  |  |  |  |  |  |
| UN2211 | POLYMERIC BEADS, EXPANDABLE, evolving flammable vapour | 9 |  | III |  | 5 |  |  | 100 |  |
| UN2212 | ASBESTOS BLUE (crocidolite) when not fixed in a natural or artificial binder material or included in a manufactured product; ASBESTOS BROWN (amosite, mysorite) when not fixed in a natural or artificial binder material or included in a manufactured product; BLUE ASBESTOS (crocidolite) when not fixed in a natural or artificial binder material or included in a manufactured product; or BROWN ASBESTOS (amosite, mysorite) when not fixed in a natural or artificial binder material or included in a manufactured product | 9 |  | II |  | 1 |  |  | Forbidden |  |
| UN2213 | PARAFORMALDEHYDE | 4.1 |  | III |  | 5 | SOR/2002-306 |  | 25 |  |
| UN2214 | PHTHALIC ANHYDRIDE with more than 0.05 per cent of maleic anhydride | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2215 | MALEIC ANHYDRIDE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2215 | MALEIC ANHYDRIDE, MOLTEN | 8 |  | III |  | 5 |  |  | Forbidden |  |
| UN2216 | FISH MEAL, STABILIZED, regulated by ship only; or FISH SCRAP, STABILIZED, regulated by ship only | 9 |  | III | 64 | 5 |  |  |  |  |
| UN2217 | SEED CAKE with not more than 1.5 per cent oil and not more than 11 per cent moisture | 4.2 |  | III | 36 | 0 |  |  | Forbidden |  |
| UN2218 | ACRYLIC ACID, STABILIZED | 8 (3) |  | II |  | 0.5 |  |  | 1 |  |
| UN2219 | ALLYL GLYCIDYL ETHER | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2222 | ANISOLE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2224 | BENZONITRILE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2225 | BENZENESULFONYL CHLORIDE; or BENZENESULPHONYL CHLORIDE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2226 | BENZOTRICHLORIDE | 8 |  | II |  | 1 | 3 000 |  | 1 |  |
| UN2227 | n-BUTYL METHACRYLATE, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2232 | 2-CHLOROETHANAL | 6.1 |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2233 | CHLOROANISIDINES | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2234 | CHLOROBENZOTRIFLUORIDES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2235 | CHLOROBENZYL CHLORIDES | 6.1 |  | III | 53 | 5 |  |  | 60 | P |
| UN2236 | 3-CHLORO-4-METHYLPHENYL ISOCYANATE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2237 | CHLORONITROANILINES | 6.1 |  | III |  | 5 |  |  | 100 | P |
| UN2238 | CHLOROTOLUENES | 3 |  | III |  | 5 |  |  | 60 | P |
| UN2239 | CHLOROTOLUIDINES, liquid; or CHLOROTOLUIDINES, LIQUID (ICAO/IMO terminology) | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2239 | CHLOROTOLUIDINES, solid; or CHLOROTOLUIDINES, SOLID (ICAO/IMO terminology) | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2240 | CHROMOSULFURIC ACID; or CHROMOSULPHURIC ACID | 8 |  | I |  | 0 | 3 000 |  | 0.5 |  |
| UN2241 | CYCLOHEPTANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2242 | CYCLOHEPTENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2243 | CYCLOHEXYL ACETATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2244 | CYCLOPENTANOL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2245 | CYCLOPENTANONE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2246 | CYCLOPENTENE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2247 | n-DECANE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2248 | DI-n-BUTYLAMINE | 8 (3) |  | II |  | 0.5 | 3 000 |  | 1 |  |
| UN2249 | DICHLORODIMETHYL ETHER, SYMMETRICAL | Forbidden |  |  |  |  |  |  |  |  |
| UN2250 | DICHLOROPHENYL ISOCYANATES | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2251 | BICYCLO[2.2.1]HEPTA-2,5-DIENE, STABILIZED; or 2,5-NORBORNADIENE, STABILIZED | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2252 | 1,2-DIMETHOXYETHANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2253 | N,N-DIMETHYLANILINE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2254 | MATCHES, FUSEE | 4.1 |  | III |  | 5 |  |  | Forbidden |  |
| UN2256 | CYCLOHEXENE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2257 | POTASSIUM | 4.3 |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2258 | 1,2-PROPYLENEDIAMINE | 8 (3) |  | II |  | 0.5 | 3 000 |  | 1 |  |
| UN2259 | TRIETHYLENETETRAMINE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2260 | TRIPROPYLAMINE | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2261 | XYLENOLS | 6.1 |  | II |  | 0.5 |  |  | 25 | P |
| UN2262 | DIMETHYLCARBAMOYL CHLORIDE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2263 | DIMETHYLCYCLOHEXANES | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2264 | DIMETHYLCYCLOHEXYLAMINE | 8 (3) |  | II |  | 0.5 |  |  | 1 |  |
| UN2265 | N,N-DIMETHYLFORMAMIDE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2266 | DIMETHYL-N-PROPYLAMINE | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2267 | DIMETHYL THIOPHOSPHORYL CHLORIDE | 6.1 (8) |  | II |  | 0.1 |  |  | 1 |  |
| UN2269 | 3,3’-IMINODIPROPYLAMINE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2270 | ETHYLAMINE, AQUEOUS SOLUTION with not less than 50 per cent but not more than 70 per cent ethylamine | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2271 | ETHYL AMYL KETONE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2272 | N-ETHYLANILINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2273 | 2-ETHYLANILINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2274 | N-ETHYL-N-BENZYLANILINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2275 | 2-ETHYLBUTANOL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2276 | 2-ETHYLHEXYLAMINE | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2277 | ETHYL METHACRYLATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2278 | n-HEPTENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2279 | HEXACHLOROBUTADIENE | 6.1 |  | III |  | 5 |  |  | 60 | PP |
| UN2280 | HEXAMETHYLENEDIAMINE, SOLID | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2281 | HEXAMETHYLENE DIISOCYANATE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2282 | HEXANOLS | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2283 | ISOBUTYL METHACRYLATE, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2284 | ISOBUTYRONITRILE | 3 (6.1) |  | II |  | 1 |  | Forbidden | 1 |  |
| UN2285 | ISOCYANATOBENZOTRIFLUORIDES | 6.1 (3) |  | II |  | 0.1 | 1 000 | Forbidden | 5 |  |
| UN2286 | PENTAMETHYLHEPTANE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2287 | ISOHEPTENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2288 | ISOHEXENE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2289 | ISOPHORONEDIAMINE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2290 | ISOPHORONE DIISOCYANATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2291 | LEAD COMPOUND, SOLUBLE, N.O.S. | 6.1 |  | III | 24 | 5 |  |  | 100 | P |
| UN2293 | 4-METHOXY-4-METHYLPENTAN-2-ONE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2294 | N-METHYLANILINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2295 | METHYL CHLOROACETATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2296 | METHYLCYCLOHEXANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2297 | METHYLCYCLOHEXANONE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2298 | METHYLCYCLOPENTANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2299 | METHYL DICHLOROACETATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2300 | 2-METHYL-5-ETHYLPYRIDINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2301 | 2-METHYLFURAN | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2302 | 5-METHYLHEXAN-2-ONE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2303 | ISOPROPENYLBENZENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2304 | NAPHTHALENE, MOLTEN | 4.1 |  | III |  | 0 |  |  | Forbidden |  |
| UN2305 | NITROBENZENESULFONIC ACID; or NITROBENZENESULPHONIC ACID | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2306 | NITROBENZOTRIFLUORIDES | 6.1 |  | II |  | 0.1 |  |  | 5 | P |
| UN2307 | 3-NITRO-4-CHLOROBENZOTRIFLUORIDE | 6.1 |  | II |  | 0.1 |  |  | 5 | P |
| UN2308 | NITROSYLSULFURIC ACID, LIQUID; or NITROSYLSULPHURIC ACID, LIQUID | 8 |  | II |  | 1 |  | Forbidden | 1 |  |
| UN2308 | NITROSYLSULFURIC ACID, SOLID; or NITROSYLSULPHURIC ACID, SOLID | 8 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2309 | OCTADIENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2310 | PENTANE-2,4-DIONE | 3 (6.1) |  | III |  | 5 |  |  | 60 |  |
| UN2311 | PHENETIDINES | 6.1 |  | III | 43 | 5 |  |  | 60 |  |
| UN2312 | PHENOL, MOLTEN | 6.1 |  | II |  | 0 | 3 000 |  | Forbidden |  |
| UN2313 | PICOLINES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2315 | ARTICLES CONTAINING POLYCHLORINATED BIPHENYLS (PCB) regulated only when the concentration is more than 50 ppm by mass; or POLYCHLORINATED BIPHENYLS (PCB) regulated only when the concentration is more than 50 ppm by mass | 9 |  | II |  | 1 |  |  | 100 | PP |
| UN2316 | SODIUM CUPROCYANIDE, SOLID | 6.1 |  | I |  | 0 | 1 000 |  | 5 | PP |
| UN2317 | SODIUM CUPROCYANIDE SOLUTION | 6.1 |  | I |  | 0 | 1 000 |  | 1 | PP |
| UN2318 | SODIUM HYDROSULFIDE with less than 25 per cent water of crystallization; or SODIUM HYDROSULPHIDE with less than 25 per cent water of crystallization | 4.2 |  | II |  | 0 |  |  | 15 |  |
| UN2319 | TERPENE HYDROCARBONS, N.O.S. | 3 |  | III |  | 5 |  |  | 60 | • |
| UN2320 | TETRAETHYLENEPENTAMINE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2321 | TRICHLOROBENZENES, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 | P |
| UN2322 | TRICHLOROBUTENE | 6.1 |  | II |  | 0.1 |  |  | 5 | P |
| UN2323 | TRIETHYL PHOSPHITE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2324 | TRIISOBUTYLENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2325 | 1,3,5-TRIMETHYLBENZENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2326 | TRIMETHYLCYCLOHEXYLAMINE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2327 | TRIMETHYLHEXAMETHYLENEDIAMINES | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2328 | TRIMETHYLHEXAMETHYLENE DIISOCYANATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2329 | TRIMETHYL PHOSPHITE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2330 | UNDECANE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2331 | ZINC CHLORIDE, ANHYDROUS | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2332 | ACETALDEHYDE OXIME | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2333 | ALLYL ACETATE | 3 (6.1) |  | II |  | 1 | 1 000 | Forbidden | 1 |  |
| UN2334 | ALLYLAMINE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2335 | ALLYL ETHYL ETHER | 3 (6.1) |  | II |  | 1 | 1 000 | Forbidden | 1 |  |
| UN2336 | ALLYL FORMATE | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2337 | PHENYL MERCAPTAN | 6.1 (3) |  | I |  | 0 | 1 000 |  | Forbidden |  |
| UN2338 | BENZOTRIFLUORIDE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2339 | 2-BROMOBUTANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2340 | 2-BROMOETHYL ETHYL ETHER | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2341 | 1-BROMO-3-METHYLBUTANE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2342 | BROMOMETHYLPROPANES | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2343 | 2-BROMOPENTANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2344 | BROMOPROPANES | 3 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2345 | 3-BROMOPROPYNE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2346 | BUTANEDIONE | 3 |  | II |  | 1 |  |  | 5 | P |
| UN2347 | BUTYL MERCAPTAN | 3 |  | II |  | 1 |  |  | 5 | P |
| UN2348 | BUTYL ACRYLATES, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2350 | BUTYL METHYL ETHER | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2351 | BUTYL NITRITES | 3 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2352 | BUTYL VINYL ETHER, STABILIZED | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2353 | BUTYRYL CHLORIDE | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2354 | CHLOROMETHYL ETHYL ETHER | 3 (6.1) |  | II |  | 1 | 3 000 | Forbidden | 1 |  |
| UN2356 | 2-CHLOROPROPANE | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
| UN2357 | CYCLOHEXYLAMINE | 8 (3) |  | II |  | 0.5 |  |  | 1 |  |
| UN2358 | CYCLOOCTATETRAENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2359 | DIALLYLAMINE | 3 (6.1) (8) |  | II |  | 1 | 1 000 |  | 1 |  |
| UN2360 | DIALLYL ETHER | 3 (6.1) |  | II |  | 1 | 1 000 | Forbidden | 1 |  |
| UN2361 | DIISOBUTYLAMINE | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2362 | 1,1-DICHLOROETHANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2363 | ETHYL MERCAPTAN | 3 |  | I |  | 0 |  | Forbidden | Forbidden | P |
| UN2364 | n-PROPYLBENZENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2366 | DIETHYL CARBONATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2367 | alpha-METHYLVALERALDEHYDE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2368 | alpha-PINENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2370 | 1-HEXENE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2371 | ISOPENTENES | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
| UN2372 | 1,2-DI-(DIMETHYLAMINO) ETHANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2373 | DIETHOXYMETHANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2374 | 3,3-DIETHOXYPROPENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2375 | DIETHYL SULFIDE; or DIETHYL SULPHIDE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2376 | 2,3-DIHYDROPYRAN | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2377 | 1,1-DIMETHOXYETHANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2378 | 2-DIMETHYLAMINOACETONITRILE | 3 (6.1) |  | II |  | 1 | 1 000 |  | 1 |  |
| UN2379 | 1,3-DIMETHYLBUTYLAMINE | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2380 | DIMETHYLDIETHOXYSILANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2381 | DIMETHYL DISULFIDE; or DIMETHYL DISULPHIDE | 3 |  | II |  | 1 | 1 000 |  | 5 |  |
| UN2382 | DIMETHYLHYDRAZINE, SYMMETRICAL | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden | P |
| UN2383 | DIPROPYLAMINE | 3 (8) |  | II |  | 1 | 3 000 |  | 1 |  |
| UN2384 | DI-n-PROPYL ETHER | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2385 | ETHYL ISOBUTYRATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2386 | 1-ETHYLPIPERIDINE | 3 (8) |  | II |  | 1 | 3 000 |  | 1 |  |
| UN2387 | FLUOROBENZENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2388 | FLUOROTOLUENES | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2389 | FURAN | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
| UN2390 | 2-IODOBUTANE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2391 | IODOMETHYLPROPANES | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2392 | IODOPROPANES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2393 | ISOBUTYL FORMATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2394 | ISOBUTYL PROPIONATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2395 | ISOBUTYRYL CHLORIDE | 3 |  | II |  | 1 | 3 000 |  | 1 |  |
|  |  | (8) |  |  |  |  |  |  |  |  |
| UN2396 | METHACRYLALDEHYDE, STABILIZED | 3 (6.1) |  | II |  | 1 | 1 000 | Forbidden | 1 |  |
| UN2397 | 3-METHYLBUTAN-2-ONE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2398 | METHYL tert-BUTYL ETHER | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2399 | 1-METHYLPIPERIDINE | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2400 | METHYL ISOVALERATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2401 | PIPERIDINE | 8 (3) |  | I |  | 0 | 3 000 | Forbidden | 0.5 |  |
| UN2402 | PROPANETHIOLS | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2403 | ISOPROPENYL ACETATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2404 | PROPIONITRILE | 3 (6.1) |  | II |  | 1 | 1 000 | Forbidden | Forbidden |  |
| UN2405 | ISOPROPYL BUTYRATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2406 | ISOPROPYL ISOBUTYRATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2407 | ISOPROPYL CHLOROFORMATE | 6.1 (3) (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2409 | ISOPROPYL PROPIONATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2410 | 1,2,3,6-TETRAHYDROPYRIDINE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2411 | BUTYRONITRILE | 3 (6.1) |  | II |  | 1 | 3 000 | Forbidden | 1 |  |
| UN2412 | TETRAHYDROTHIOPHENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2413 | TETRAPROPYL ORTHOTITANATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2414 | THIOPHENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2416 | TRIMETHYL BORATE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2417 | CARBONYL FLUORIDE, COMPRESSED | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2418 | SULFUR TETRAFLUORIDE; or SULPHUR TETRAFLUORIDE | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2419 | BROMOTRIFLUOROETHYLENE | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | Forbidden |  |
| UN2420 | HEXAFLUOROACETONE | 2.3 (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2421 | NITROGEN TRIOXIDE | 2.3 (5.1) (8) |  |  | 38 | 0 | 25 SOR/2002-306 | Forbidden | Forbidden |  |
| UN2422 | OCTAFLUOROBUT-2-ENE; or REFRIGERANT GAS R 1318 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN2424 | OCTAFLUOROPROPANE; or REFRIGERANT GAS R 218 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN2426 | AMMONIUM NITRATE, LIQUID (hot concentrated solution) with not more than 0.2 per cent combustible material, in a concentration exceeding 80 per cent | 5.1 |  |  |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2427 | POTASSIUM CHLORATE, AQUEOUS SOLUTION | 5.1 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 2.5 |  |
| UN2428 | SODIUM CHLORATE, AQUEOUS SOLUTION | 5.1 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 2.5 |  |
| UN2429 | CALCIUM CHLORATE, AQUEOUS SOLUTION | 5.1 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 2.5 |  |
| UN2430 | ALKYLPHENOLS, SOLID, N.O.S. including C2-C12 homologues | 8 |  | I | 50 | 0 | 3 000 |  | 1 | • |
|  |  |  |  | II | 50 | 1 |  |  | 15 | • |
|  |  |  |  | III | 50 | 5 |  |  | 25 | • |
| UN2431 | ANISIDINES, liquid; or ANISIDINES, LIQUID (ICAO/IMO terminology) | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2431 | ANISIDINES, solid; or ANISIDINES, SOLID (ICAO/IMO terminology) | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2432 | N,N-DIETHYLANILINE | 6.1 |  | III | 43 | 5 |  |  | 60 |  |
| UN2433 | CHLORONITROTOLUENES, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 | P |
| UN2433 | CHLORONITROTOLUENES, SOLID | 6.1 |  | III |  | 5 |  |  | 100 | P |
| UN2434 | DIBENZYLDICHLOROSILANE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2435 | ETHYLPHENYLDICHLOROSILANE | 8 |  | II |  | 1 |  |  | Forbidden |  |
| UN2436 | THIOACETIC ACID | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2437 | METHYLPHENYLDICHLOROSILANE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2438 | TRIMETHYLACETYL CHLORIDE | 6.1 (3) (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2439 | SODIUM HYDROGENDIFLUORIDE, solution; or SODIUM HYDROGENDIFLUORIDE, SOLUTION (ICAO terminology) | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2439 | SODIUM HYDROGENDIFLUORIDE, solid; or SODIUM HYDROGENDIFLUORIDE, SOLID (ICAO terminology) | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2440 | STANNIC CHLORIDE PENTAHYDRATE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2441 | TITANIUM TRICHLORIDE MIXTURE, PYROPHORIC; or TITANIUM TRICHLORIDE, PYROPHORIC | 4.2 (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2442 | TRICHLOROACETYL CHLORIDE | 8 |  | II |  | 1 |  | Forbidden | Forbidden |  |
| UN2443 | VANADIUM OXYTRICHLORIDE | 8 |  | II |  | 1 |  |  | Forbidden |  |
| UN2444 | VANADIUM TETRACHLORIDE | 8 |  | I |  | 0 | 3 000 |  | Forbidden |  |
| UN2445 | LITHIUM ALKYLS | 4.2 (4.3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2446 | NITROCRESOLS | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2447 | PHOSPHORUS, WHITE, MOLTEN | 4.2 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden | PP |
| UN2448 | MOLTEN SULFUR; MOLTEN SULPHUR; SULFUR, MOLTEN; or SULPHUR, MOLTEN | 4.1 |  | III | 32 | 0 |  |  | Forbidden |  |
| UN2451 | NITROGEN TRIFLUORIDE, COMPRESSED | 2.2 (5.1) |  |  | 38 | 0 | 25 | Forbidden | 75 |  |
| UN2452 | ETHYLACETYLENE, STABILIZED | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | Forbidden |  |
| UN2453 | ETHYL FLUORIDE; or REFRIGERANT GAS R 161 | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN2454 | METHYL FLUORIDE; or REFRIGERANT GAS R 41 | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN2455 | METHYL NITRITE | Forbidden |  |  |  |  |  |  |  |  |
| UN2456 | 2-CHLOROPROPENE | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
| UN2457 | 2,3-DIMETHYLBUTANE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2458 | HEXADIENE | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2459 | 2-METHYL-1-BUTENE | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
| UN2460 | 2-METHYL-2-BUTENE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2461 | METHYLPENTADIENE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2463 | ALUMINUM HYDRIDE | 4.3 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2464 | BERYLLIUM NITRATE | 5.1 (6.1) |  | II |  | 0.5 | 1 000 |  | 5 |  |
| UN2465 | DICHLOROISOCYANURIC ACID, DRY; or DICHLOROISOCYANURIC ACID SALTS excluding the dihydrated sodium salts | 5.1 |  | II |  | 1 |  |  | 5 |  |
| UN2466 | POTASSIUM SUPEROXIDE | 5.1 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2468 | TRICHLOROISOCYANURIC ACID, DRY | 5.1 |  | II |  | 1 |  |  | 5 |  |
| UN2469 | ZINC BROMATE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2470 | PHENYLACETONITRILE, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2471 | OSMIUM TETROXIDE | 6.1 |  | I | 38 | 0 | 1 000 |  | 5 | PP |
| UN2473 | SODIUM ARSANILATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2474 | THIOPHOSGENE | 6.1 |  | II | 43 | 0.1 | 1 000 |  | Forbidden |  |
| UN2475 | VANADIUM TRICHLORIDE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2477 | METHYL ISOTHIOCYANATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2478 | ISOCYANATES, FLAMMABLE, TOXIC, N.O.S.; or ISOCYANATE SOLUTION, FLAMMABLE, TOXIC, N.O.S. | 3 (6.1) |  | II | 16 | 1 | 1 000 | Forbidden | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  |  | • |
| UN2480 | METHYL ISOCYANATE | 6.1 (3) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2481 | ETHYL ISOCYANATE | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2482 | n-PROPYL ISOCYANATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2483 | ISOPROPYL ISOCYANATE | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2484 | tert-BUTYL ISOCYANATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2485 | n-BUTYL ISOCYANATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2486 | ISOBUTYL ISOCYANATE | 3 (6.1) |  | II |  | 1 | 1 000 | Forbidden | 1 |  |
| UN2487 | PHENYL ISOCYANATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2488 | CYCLOHEXYL ISOCYANATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2490 | DICHLOROISOPROPYL ETHER | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2491 | ETHANOLAMINE; or ETHANOLAMINE SOLUTION | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2493 | HEXAMETHYLENEIMINE | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2495 | IODINE PENTAFLUORIDE | 5.1 (6.1) (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2496 | PROPIONIC ANHYDRIDE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2498 | 1,2,3,6-TETRAHYDROBENZALDEHYDE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2501 | TRIS-(1-AZIRIDINYL) PHOSPHINE OXIDE SOLUTION | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2502 | VALERYL CHLORIDE | 8 (3) |  | II |  | 0.5 |  |  | 1 |  |
| UN2503 | ZIRCONIUM TETRACHLORIDE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2504 | TETRABROMOETHANE | 6.1 |  | III |  | 5 |  |  | 60 | P |
| UN2505 | AMMONIUM FLUORIDE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2506 | AMMONIUM HYDROGEN SULFATE; or AMMONIUM HYDROGEN SULPHATE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2507 | CHLOROPLATINIC ACID, SOLID | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2508 | MOLYBDENUM PENTACHLORIDE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2509 | POTASSIUM HYDROGEN SULFATE; or POTASSIUM HYDROGEN SULPHATE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2511 | 2-CHLOROPROPIONIC ACID, SOLUTION | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2511 | 2-CHLOROPROPIONIC ACID, SOLID | 6.1 |  | III |  | 5 |  |  | 25 |  |
| UN2512 | AMINOPHENOLS (o-,m-,p-) | 6.1 |  | III | 43 | 5 |  |  | 100 |  |
| UN2513 | BROMOACETYL BROMIDE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2514 | BROMOBENZENE | 3 |  | III |  | 5 |  |  | 60 | P |
| UN2515 | BROMOFORM | 6.1 |  | III |  | 5 |  |  | 60 | P |
| UN2516 | CARBON TETRABROMIDE | 6.1 |  | III |  | 5 |  |  | 100 | P |
| UN2517 | 1-CHLORO-1,1-DIFLUOROETHANE; or REFRIGERANT GAS R 142b | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | Forbidden |  |
| UN2518 | 1,5,9-CYCLODODECATRIENE | 6.1 |  | III |  | 5 |  |  | 60 | PP |
| UN2520 | CYCLOOCTADIENES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2521 | DIKETENE, STABILIZED | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2522 | 2-DIMETHYLAMINOETHYL METHACRYLATE | 6.1 |  | II |  | 0.1 |  | Forbidden | 5 |  |
| UN2524 | ETHYL ORTHOFORMATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2525 | ETHYL OXALATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2526 | FURFURYLAMINE | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2527 | ISOBUTYL ACRYLATE, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2528 | ISOBUTYL ISOBUTYRATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2529 | ISOBUTYRIC ACID | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2531 | METHACRYLIC ACID, STABILIZED | 8 |  | II |  | 1 |  |  | 5 |  |
| UN2533 | METHYL TRICHLOROACETATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2534 | METHYLCHLOROSILANE | 2.3 (2.1) (8) |  |  |  | 0 | 25 | Forbidden | Forbidden |  |
| UN2535 | 4-METHYLMORPHOLINE; or N-METHYLMORPHOLINE | 3 (8) |  | II |  | 1 | 3 000 |  | 1 |  |
| UN2536 | METHYLTETRAHYDROFURAN | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2538 | NITRONAPHTHALENE | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2541 | TERPINOLENE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2542 | TRIBUTYLAMINE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2545 | HAFNIUM POWDER, DRY | 4.2 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
|  |  |  |  | II |  | 0 |  | Forbidden | 15 |  |
|  |  |  |  | III |  | 0 |  | Forbidden | 25 |  |
| UN2546 | TITANIUM POWDER, DRY | 4.2 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
|  |  |  |  | II |  | 0 |  | Forbidden | 15 |  |
|  |  |  |  | III |  | 0 |  | Forbidden | 25 |  |
| UN2547 | SODIUM SUPEROXIDE | 5.1 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2548 | CHLORINE PENTAFLUORIDE | 2.3 (5.1) (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2552 | HEXAFLUOROACETONE HYDRATE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2554 | METHYLALLYL CHLORIDE | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2555 | NITROCELLULOSE WITH WATER, not less than 25 per cent water, by mass | 4.1 |  | II | 38 | 0 |  | Forbidden | 15 |  |
| UN2556 | NITROCELLULOSE WITH ALCOHOL, not less than 25 per cent alcohol, by mass, and not more than 12.6 per cent nitrogen, by dry mass | 4.1 |  | II | 38 | 0 |  | Forbidden | 1 |  |
| UN2557 | NITROCELLULOSE, MIXTURE, WITHOUT PIGMENT with not more than 12.6 per cent nitrogen, by dry mass; NITROCELLULOSE, MIXTURE, WITHOUT PLASTICIZER with not more than 12.6 per cent nitrogen, by dry mass; NITROCELLULOSE, MIXTURE, WITH PIGMENT with not more than 12.6 per cent nitrogen, by dry mass; or NITROCELLULOSE, MIXTURE, WITH PLASTICIZER with not more than 12.6 per cent nitrogen, by dry mass | 4.1 |  | II | 38, 70 | 0 |  | Forbidden | 1 | • |
| UN2558 | EPIBROMOHYDRIN | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden | P |
| UN2560 | 2-METHYLPENTAN-2-OL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2561 | 3-METHYL-1-BUTENE | 3 |  | I |  | 0 |  | Forbidden | 1 |  |
| UN2564 | TRICHLOROACETIC ACID SOLUTION | 8 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 5 |  |
| UN2565 | DICYCLOHEXYLAMINE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2567 | SODIUM PENTACHLOROPHENATE | 6.1 |  | II |  | 0.5 |  |  | 25 | PP |
| UN2570 | CADMIUM COMPOUND | 6.1 |  | I | 23, 51 | 0 | 1 000 |  | 5 | PP |
|  |  |  |  | II |  | 0.5 |  |  | 25 | PP |
|  |  |  |  | III |  | 5 |  |  | 100 | PP |
| UN2571 | ALKYLSULFURIC ACIDS; or ALKYLSULPHURIC ACIDS | 8 |  | II | 16 | 1 |  |  | 1 |  |
| UN2572 | PHENYLHYDRAZINE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2573 | THALLIUM CHLORATE | 5.1 (6.1) |  | II |  | 0.5 | 1 000 |  | 5 | P |
| UN2574 | TRICRESYL PHOSPHATE with more than 3 per cent ortho isomer | 6.1 |  | II |  | 0.1 |  |  | 5 | PP |
| UN2576 | PHOSPHORUS OXYBROMIDE, MOLTEN | 8 |  | II |  | 1 |  |  | Forbidden |  |
| UN2577 | PHENYLACETYL CHLORIDE | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2578 | PHOSPHORUS TRIOXIDE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2579 | PIPERAZINE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2580 | ALUMINUM BROMIDE SOLUTION | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2581 | ALUMINUM CHLORIDE SOLUTION | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2582 | FERRIC CHLORIDE SOLUTION | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2583 | ALKYLSULFONIC ACIDS, SOLID with more than 5 per cent free sulfuric acid; ALKYLSULPHONIC ACIDS, SOLID with more than 5 per cent free sulphuric acid; ARYLSULFONIC ACIDS, SOLID with more than 5 per cent free sulfuric acid; or ARYLSULPHONIC ACIDS, SOLID with more than 5 per cent free sulphuric acid | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2584 | ALKYLSULFONIC ACIDS, LIQUID with more than 5 per cent free sulfuric acid; ALKYLSULPHONIC ACIDS, LIQUID with more than 5 per cent free sulphuric acid; ARYLSULFONIC ACIDS, LIQUID with more than 5 per cent free sulfuric acid; or ARYLSULPHONIC ACIDS, LIQUID with more than 5 per cent free sulphuric acid | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2585 | ALKYLSULFONIC ACIDS, SOLID with not more than 5 per cent free sulfuric acid; ALKYLSULPHONIC ACIDS, SOLID with not more than 5 per cent free sulphuric acid; ARYLSULFONIC ACIDS, SOLID with not more than 5 per cent free sulfuric acid; or ARYLSULPHONIC ACIDS, SOLID with not more than 5 per cent free sulphuric acid | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2586 | ALKYLSULFONIC ACIDS, LIQUID with not more than 5 per cent free sulfuric acid; ALKYLSULPHONIC ACIDS, LIQUID with not more than 5 per cent free sulphuric acid; ARYLSULFONIC ACIDS, LIQUID with not more than 5 per cent free sulfuric acid; or ARYLSULPHONIC ACIDS, LIQUID with not more than 5 per cent free sulphuric acid | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2587 | BENZOQUINONE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2588 | PESTICIDE, SOLID, TOXIC, N.O.S. | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2589 | VINYL CHLOROACETATE | 6.1 (3) |  | II |  | 0.1 | 1 000 |  | 5 |  |
| UN2590 | ASBESTOS, WHITE (chrysotile, actinolite, anthophyllite, tremolite) when not fixed in a natural or artificial binder material or included in a manufactured product | 9 |  | III |  | 5 |  |  | 200 |  |
| UN2591 | XENON, REFRIGERATED LIQUID | 2.2 |  |  | SOR/2002-306 | 0.125 SOR/2002-306 |  |  | 50 |  |
| UN2599 | CHLOROTRIFLUOROMETHANE AND TRIFLUOROMETHANE AZEOTROPIC MIXTURE with approximately 60 per cent chlorotrifluoromethane; or REFRIGERANT GAS R 503 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN2600 | CARBON MONOXIDE AND HYDROGEN MIXTURE, COMPRESSED | 2.3 (2.1) |  |  |  | 0 | 500 | Forbidden | Forbidden |  |
| UN2601 | CYCLOBUTANE | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | Forbidden |  |
| UN2602 | DICHLORODIFLUOROMETHANE AND DIFLUOROETHANE AZEOTROPIC MIXTURE with approximately 74 per cent dichlorodifluoromethane; or REFRIGERANT GAS R 500 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN2603 | CYCLOHEPTATRIENE | 3 (6.1) |  | II |  | 1 |  | Forbidden | 1 |  |
| UN2604 | BORON TRIFLUORIDE DIETHYL ETHERATE | 8 (3) |  | I |  | 0 | 3 000 | Forbidden | 0.5 |  |
| UN2605 | METHOXYMETHYL ISOCYANATE | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2606 | METHYL ORTHOSILICATE | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2607 | ACROLEIN DIMER, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2608 | NITROPROPANES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2609 | TRIALLYL BORATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2610 | TRIALLYLAMINE | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2611 | PROPYLENE CHLOROHYDRIN | 6.1 (3) |  | II |  | 0.1 | 1 000 |  | 5 |  |
| UN2612 | METHYL PROPYL ETHER | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2614 | METHALLYL ALCOHOL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2615 | ETHYL PROPYL ETHER | 3 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2616 | TRIISOPROPYL BORATE | 3 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2617 | METHYLCYCLOHEXANOLS, flammable | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2618 | VINYLTOLUENES, STABILIZED | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2619 | BENZYLDIMETHYLAMINE | 8 |  | II |  | 0.5 |  |  | 1 |  |
|  |  | (3) |  |  |  |  |  |  |  |  |
| UN2620 | AMYL BUTYRATES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2621 | ACETYL METHYL CARBINOL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2622 | GLYCIDALDEHYDE | 3 (6.1) |  | II |  | 1 | 1 000 |  | 1 |  |
| UN2623 | FIRELIGHTERS, SOLID with flammable liquid | 4.1 |  | III |  | 5 |  |  | 15 | • |
| UN2624 | MAGNESIUM SILICIDE | 4.3 |  | II |  | 0.5 |  |  | 15 |  |
| UN2626 | CHLORIC ACID, AQUEOUS SOLUTION with not more than 10 per cent chloric acid | 5.1 |  | II | 38, 68 | 1 |  | Forbidden | Forbidden |  |
| UN2627 | NITRITES, INORGANIC, N.O.S. | 5.1 |  | II | 38, 68, 71 | 1 |  |  | 5 | • |
| UN2628 | POTASSIUM FLUOROACETATE | 6.1 |  | I |  | 0 | 1 000 | Forbidden | 5 |  |
| UN2629 | SODIUM FLUOROACETATE | 6.1 |  | I |  | 0 | 1 000 | Forbidden | 5 |  |
| UN2630 | SELENATES; or SELENITES | 6.1 |  | I |  | 0 | 1 000 | Forbidden | 5 | • |
| UN2642 | FLUOROACETIC ACID | 6.1 |  | I |  | 0 | 1 000 | Forbidden | 1 |  |
| UN2643 | METHYL BROMOACETATE | 6.1 |  | II |  | 0.1 |  | Forbidden | 5 |  |
| UN2644 | METHYL IODIDE | 6.1 |  | I |  | 0 | 1 000 |  | Forbidden |  |
| UN2645 | PHENACYL BROMIDE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2646 | HEXACHLOROCYCLOPENTADIENE | 6.1 |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2647 | MALONONITRILE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2648 | 1,2-DIBROMOBUTAN-3-ONE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2649 | 1,3-DICHLOROACETONE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2650 | 1,1-DICHLORO-1-NITROETHANE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2651 | 4,4’-DIAMINODIPHENYLMETHANE | 6.1 |  | III |  | 5 |  |  | 100 | P |
| UN2653 | BENZYL IODIDE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2655 | POTASSIUM FLUOROSILICATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2656 | QUINOLINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2657 | SELENIUM DISULFIDE; or SELENIUM DISULPHIDE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2659 | SODIUM CHLOROACETATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2660 | MONONITROTOLUIDINES; or NITROTOLUIDINES | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2661 | HEXACHLOROACETONE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2662 | HYDROQUINONE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2664 | DIBROMOMETHANE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2667 | BUTYLTOLUENES | 6.1 |  | III |  | 5 |  |  | 60 | • |
| UN2668 | CHLOROACETONITRILE | 6.1 |  | II |  | 0.1 | 1 000 |  | Forbidden |  |
|  |  | (3) |  |  |  |  |  |  |  |  |
| UN2669 | CHLOROCRESOLS, liquid; CHLOROCRESOLS, LIQUID (ICAO terminology); or CHLOROCRESOLS, SOLUTION (IMO terminology) | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2669 | CHLOROCRESOLS, solid; or CHLOROCRESOLS, SOLID (ICAO terminology) | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2670 | CYANURIC CHLORIDE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2671 | AMINOPYRIDINES (o-,m-,p-) | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2672 | AMMONIA SOLUTION, relative density between 0.880 and 0.957 at 15 °C in water, with more than 10 per cent but not more than 35 per cent ammonia | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2673 | 2-AMINO-4-CHLOROPHENOL | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2674 | SODIUM FLUOROSILICATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2676 | STIBINE | 2.3 (2.1) |  |  | 38 | 0 | 0 | Forbidden | Forbidden |  |
| UN2677 | RUBIDIUM HYDROXIDE SOLUTION | 8 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 5 |  |
| UN2678 | RUBIDIUM HYDROXIDE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2679 | LITHIUM HYDROXIDE SOLUTION | 8 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 5 |  |
| UN2680 | LITHIUM HYDROXIDE MONOHYDRATE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2681 | CAESIUM HYDROXIDE SOLUTION | 8 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 5 |  |
| UN2682 | CAESIUM HYDROXIDE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2683 | AMMONIUM SULFIDE SOLUTION; or AMMONIUM SULPHIDE SOLUTION | 8 (3) (6.1) |  | II |  | 0.5 | 1 000 |  | 1 |  |
| UN2684 | DIETHYLAMINOPROPYLAMINE | 3 (8) |  | III |  | 5 |  |  | 5 |  |
| UN2685 | N,N-DIETHYLETHYLENEDIAMINE | 8 (3) |  | II |  | 0.5 |  |  | 1 |  |
| UN2686 | 2-DIETHYLAMINOETHANOL | 8 (3) |  | II |  | 0.5 |  |  | 1 |  |
| UN2687 | DICYCLOHEXYLAMMONIUM NITRITE | 4.1 |  | III |  | 5 |  |  | 25 | P |
| UN2688 | 1-BROMO-3-CHLOROPROPANE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2689 | GLYCEROL alpha-MONOCHLOROHYDRIN | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2690 | N,n-BUTYLIMIDAZOLE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2691 | PHOSPHORUS PENTABROMIDE | 8 |  | II |  | 1 |  |  | Forbidden |  |
| UN2692 | BORON TRIBROMIDE | 8 |  | I |  | 0 | 1 000 |  | Forbidden |  |
| UN2693 | BISULFITES, AQUEOUS SOLUTION, N.O.S.; or BISULPHITES, AQUEOUS SOLUTION, N.O.S. | 8 |  | III | 16 | 5 |  |  | 5 | • |
| UN2698 | TETRAHYDROPHTHALIC ANHYDRIDES with more than 0.05 per cent of maleic anhydride | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2699 | TRIFLUOROACETIC ACID | 8 |  | I |  | 0 | 3 000 |  | 0.5 |  |
| UN2705 | 1-PENTOL | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2707 | DIMETHYLDIOXANES | 3 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2709 | BUTYLBENZENES | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2710 | DIPROPYL KETONE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2713 | ACRIDINE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2714 | ZINC RESINATE | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2715 | ALUMINUM RESINATE | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2716 | 1,4-BUTYNEDIOL | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2717 | CAMPHOR, synthetic | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2719 | BARIUM BROMATE | 5.1 (6.1) |  | II |  | 0.5 | 1 000 |  | 5 |  |
| UN2720 | CHROMIUM NITRATE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2721 | COPPER CHLORATE | 5.1 |  | II |  | 1 |  |  | 5 |  |
| UN2722 | LITHIUM NITRATE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2723 | MAGNESIUM CHLORATE | 5.1 |  | II |  | 1 |  |  | 5 |  |
| UN2724 | MANGANESE NITRATE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2725 | NICKEL NITRATE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2726 | NICKEL NITRITE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2727 | THALLIUM NITRATE | 6.1 (5.1) |  | II |  | 0.5 |  |  | 5 | P |
| UN2728 | ZIRCONIUM NITRATE | 5.1 |  | III |  | 5 |  |  | 25 |  |
| UN2729 | HEXACHLOROBENZENE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2730 | NITROANISOLES, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2730 | NITROANISOLES, SOLID | 6.1 |  | III | 43 | 5 |  |  | 100 |  |
| UN2732 | NITROBROMOBENZENES, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2732 | NITROBROMOBENZENES, SOLID | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2733 | AMINES, FLAMMABLE, CORROSIVE, N.O.S.; or POLYAMINES, FLAMMABLE, CORROSIVE, N.O.S. | 3 (8) |  | I | 16 | 0 | 1 000 | Forbidden | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN2734 | AMINES, LIQUID, CORROSIVE, FLAMMABLE, N.O.S.; or POLYAMINES, LIQUID, CORROSIVE, FLAMMABLE, N.O.S. | 8 (3) |  | I | 16 | 0 | 1 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 1 | • |
| UN2735 | AMINES, LIQUID, CORROSIVE, N.O.S.; or POLYAMINES, LIQUID, CORROSIVE, N.O.S. | 8 |  | I | 16 | 0 | 1 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN2738 | N-BUTYLANILINE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2739 | BUTYRIC ANHYDRIDE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2740 | n-PROPYL CHLOROFORMATE | 6.1 (3) (8) |  | I |  | 0 | 1 000 |  | Forbidden |  |
| UN2741 | BARIUM HYPOCHLORITE with more than 22 per cent available chlorine | 5.1 (6.1) |  | II | 38 | 0.5 | 1 000 |  | 5 |  |
| UN2742 | CHLOROFORMATES, TOXIC, CORROSIVE, FLAMMABLE, N.O.S. | 6.1 (3) (8) |  | II |  | 0.1 | 1 000 |  | 1 | • |
| UN2743 | n-BUTYL CHLOROFORMATE | 6.1 (3) (8) |  | II |  | 0.1 | 1 000 |  | 1 |  |
| UN2744 | CYCLOBUTYL CHLOROFORMATE | 6.1 (3) (8) |  | II |  | 0.1 | 1 000 |  | 1 |  |
| UN2745 | CHLOROMETHYL CHLOROFORMATE | 6.1 (8) |  | II |  | 0.1 | 1 000 |  | 1 |  |
| UN2746 | PHENYL CHLOROFORMATE | 6.1 (8) |  | II |  | 0.1 | 1 000 |  | 1 |  |
| UN2747 | tert-BUTYLCYCLOHEXYL CHLOROFORMATE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2748 | 2-ETHYLHEXYL CHLOROFORMATE | 6.1 (8) |  | II |  | 0.1 | 3 000 |  | 1 |  |
| UN2749 | TETRAMETHYLSILANE | 3 |  | I |  | 0 |  | Forbidden | Forbidden |  |
| UN2750 | 1,3-DICHLOROPROPANOL-2 | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2751 | DIETHYLTHIOPHOSPHORYL CHLORIDE | 8 |  | II |  | 1 |  | Forbidden | 15 |  |
| UN2752 | 1,2-EPOXY-3-ETHOXYPROPANE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2753 | N-ETHYLBENZYLTOLUIDINES, LIQUID | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2753 | N-ETHYLBENZYLTOLUIDINES, SOLID | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2754 | N-ETHYLTOLUIDINES | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2757 | CARBAMATE PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2758 | CARBAMATE PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2759 | ARSENICAL PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2760 | ARSENICAL PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2761 | ORGANOCHLORINE PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2762 | ORGANOCHLORINE PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2763 | TRIAZINE PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2764 | TRIAZINE PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2771 | THIOCARBAMATE PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2772 | THIOCARBAMATE PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2775 | COPPER BASED PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2776 | COPPER BASED PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2777 | MERCURY BASED PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | PP |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | PP |
|  |  |  |  | III | 16 | 5 |  |  | 100 | PP |
| UN2778 | MERCURY BASED PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | PP |
|  |  |  |  | II | 16 | 1 |  |  | 1 | PP |
| UN2779 | SUBSTITUTED NITROPHENOL PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2780 | SUBSTITUTED NITROPHENOL PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2781 | BIPYRIDILIUM PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2782 | BIPYRIDILIUM PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2783 | ORGANOPHOSPHORUS PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2784 | ORGANOPHOSPHORUS PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16, 23 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN2785 | 4-THIAPENTANAL | 6.1 |  | III |  | 5 |  | Forbidden | 60 |  |
| UN2786 | ORGANOTIN PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | PP |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | PP |
|  |  |  |  | III | 16 | 5 |  |  | 100 | PP |
| UN2787 | ORGANOTIN PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | PP |
|  |  |  |  | II | 16 | 1 |  |  | 1 | PP |
| UN2788 | ORGANOTIN COMPOUND, LIQUID, N.O.S. | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | PP |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | PP |
|  |  |  |  | III | 16 | 5 |  |  | 60 | PP |
| UN2789 | ACETIC ACID, GLACIAL; or ACETIC ACID SOLUTION, more than 80 per cent acid, by mass | 8 (3) |  | II |  | 0.5 | 3 000 |  | 1 |  |
| UN2790 | ACETIC ACID SOLUTION, more than 10 per cent and less than 50 per cent acid, by mass | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2790 | ACETIC ACID SOLUTION, not less than 50 per cent but not more than 80 per cent acid, by mass | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2793 | FERROUS METAL BORINGS, SHAVINGS, TURNINGS or CUTTINGS in a form liable to self-heating | 4.2 |  | III |  | 0 |  |  | 25 |  |
| UN2794 | BATTERIES, WET, FILLED WITH ACID, electric storage | 8 |  | III | 39 | 5 |  |  | 30 |  |
| UN2795 | BATTERIES, WET, FILLED WITH ALKALI, electric storage | 8 |  | III | 39 | 5 |  |  | 30 |  |
| UN2796 | BATTERY FLUID, ACID; SULFURIC ACID with not more than 51 per cent acid; or SULPHURIC ACID with not more than 51 per cent acid | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2797 | BATTERY FLUID, ALKALI | 8 |  | II |  | 1 |  |  | 1 |  |
| UN2798 | PHENYLPHOSPHORUS DICHLORIDE | 8 |  | II |  | 1 |  |  | Forbidden |  |
| UN2799 | PHENYLPHOSPHORUS THIODICHLORIDE | 8 |  | II |  | 1 |  |  | Forbidden |  |
| UN2800 | BATTERIES, WET, NON-SPILLABLE, electric storage | 8 |  | III | 39 | 5 |  |  |  |  |
| UN2801 | DYE INTERMEDIATE, LIQUID, CORROSIVE, N.O.S.; or DYE, LIQUID, CORROSIVE, N.O.S. | 8 |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN2802 | COPPER CHLORIDE | 8 |  | III |  | 5 |  |  | 25 | PP |
| UN2803 | GALLIUM | 8 |  | III |  | 5 |  |  | 20 |  |
| UN2805 | LITHIUM HYDRIDE, FUSED SOLID | 4.3 |  | II |  | 0.5 |  | Forbidden | 15 |  |
| UN2806 | LITHIUM NITRIDE | 4.3 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2807 | MAGNETIZED MATERIAL, regulated by aircraft only | 9 |  | III | 63 | 5 |  |  |  |  |
| UN2809 | MERCURY | 8 |  | III |  | 5 |  |  | 35 |  |
| UN2810 | TOXIC LIQUID, ORGANIC, N.O.S. | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN2811 | TOXIC SOLID, ORGANIC, N.O.S. | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN2812 | SODIUM ALUMINATE, SOLID, regulated by aircraft only | 8 |  | III | 63 | 5 |  |  | 25 |  |
| UN2813 | WATER-REACTIVE SOLID, N.O.S. | 4.3 |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 25 | • |
| UN2814 | INFECTIOUS SUBSTANCE, AFFECTING HUMANS | 6.2 |  | (RG 4) | 38 | 0 | 0 | 0.05 | 0.05 |  |
|  |  |  |  | (RG 3) | 38 | 0 |  | 0.05 | 0.05 |  |
|  |  |  |  | (RG 2) |  | 0 |  |  |  |  |
| UN2815 | N-AMINOETHYLPIPERAZINE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2817 | AMMONIUM HYDROGENDIFLUORIDE SOLUTION | 8 (6.1) |  | II |  | 0.5 | 3 000 |  | 1 |  |
|  |  |  |  | III |  | 1 |  |  | 5 |  |
| UN2818 | AMMONIUM POLYSULFIDE SOLUTION; or AMMONIUM POLYSULPHIDE SOLUTION | 8 (6.1) |  | II |  | 0.5 | 3 000 |  | 1 |  |
|  |  |  |  | III |  | 1 |  |  | 5 |  |
| UN2819 | AMYL ACID PHOSPHATE | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2820 | BUTYRIC ACID | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2821 | PHENOL SOLUTION | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2822 | 2-CHLOROPYRIDINE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2823 | CROTONIC ACID, liquid; or CROTONIC ACID, LIQUID (ICAO terminology) | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2823 | CROTONIC ACID, solid; or CROTONIC ACID, SOLID (ICAO terminology) | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2826 | ETHYL CHLOROTHIOFORMATE | 8 (3) |  | II |  | 0 | 1 000 |  | Forbidden | P |
| UN2829 | CAPROIC ACID | 8 |  | III |  | 5 |  |  | 5 |  |
| UN2830 | LITHIUM FERROSILICON | 4.3 |  | II |  | 0.5 | 3 000 | Forbidden | 15 |  |
| UN2831 | 1,1,1-TRICHLOROETHANE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2834 | PHOSPHOROUS ACID | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2835 | SODIUM ALUMINIUM HYDRIDE | 4.3 |  | II |  | 0.5 |  | Forbidden | Forbidden |  |
| UN2837 | BISULFATES, AQUEOUS SOLUTION; or BISULPHATES, AQUEOUS SOLUTION | 8 |  | II |  | 1 |  |  | 1 | • |
|  |  |  |  | III |  | 5 |  |  | 5 | • |
| UN2838 | VINYL BUTYRATE, STABILIZED | 3 |  | II |  | 1 |  |  | 5 |  |
| UN2839 | ALDOL | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2840 | BUTYRALDOXIME | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2841 | DI-n-AMYLAMINE | 3 (6.1) |  | III |  | 5 |  |  | 60 |  |
| UN2842 | NITROETHANE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2844 | CALCIUM MANGANESE SILICON | 4.3 |  | III |  | 1 |  |  | 25 |  |
| UN2845 | PYROPHORIC LIQUID, ORGANIC, N.O.S. | 4.2 |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN2846 | PYROPHORIC SOLID, ORGANIC, N.O.S. | 4.2 |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN2849 | 3-CHLOROPROPANOL-1 | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2850 | PROPYLENE TETRAMER | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2851 | BORON TRIFLUORIDE DIHYDRATE | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2852 | DIPICRYL SULFIDE, WETTED with not less than 10 per cent water, by mass; or DIPICRYL SULPHIDE, WETTED with not less than 10 per cent water, by mass | 4.1 |  | I | 38 | 0 | 75 | Forbidden | Forbidden |  |
| UN2853 | MAGNESIUM FLUOROSILICATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2854 | AMMONIUM FLUOROSILICATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2855 | ZINC FLUOROSILICATE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2856 | FLUOROSILICATES, N.O.S. | 6.1 |  | III |  | 5 |  |  | 100 | • |
| UN2857 | REFRIGERATING MACHINES containing non-flammable, non-toxic, liquefied gas or ammonia solutions (UN2672) | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  |  |  |
| UN2858 | ZIRCONIUM, DRY, coiled wire, finished metal sheets, strip (thinner than 254 microns but not thinner than 18 microns) | 4.1 |  | III |  | 5 |  |  | 25 |  |
| UN2859 | AMMONIUM METAVANADATE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2861 | AMMONIUM POLYVANADATE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2862 | VANADIUM PENTOXIDE, non-fused form | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2863 | SODIUM AMMONIUM VANADATE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2864 | POTASSIUM METAVANADATE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2865 | HYDROXYLAMINE SULFATE; or HYDROXYLAMINE SULPHATE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2869 | TITANIUM TRICHLORIDE MIXTURE | 8 |  | II |  | 1 |  |  | 15 |  |
|  |  |  |  | III |  | 5 |  |  | 25 |  |
| UN2870 | ALUMINUM BOROHYDRIDE; or ALUMINUM BOROHYDRIDE IN DEVICES | 4.2 (4.3) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2871 | ANTIMONY POWDER | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2872 | DIBROMOCHLOROPROPANES | 6.1 |  | II |  | 0.1 | 1 000 |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN2873 | DIBUTYLAMINOETHANOL | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2874 | FURFURYL ALCOHOL | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2875 | HEXACHLOROPHENE | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2876 | RESORCINOL | 6.1 |  | III |  | 5 |  |  | 100 |  |
| UN2878 | TITANIUM SPONGE GRANULES; or TITANIUM SPONGE POWDERS | 4.1 |  | III |  | 5 |  | Forbidden | 25 |  |
| UN2879 | SELENIUM OXYCHLORIDE | 8 (6.1) |  | I |  | 0 | 1 000 | Forbidden | 0.5 |  |
| UN2880 | CALCIUM HYPOCHLORITE, HYDRATED MIXTURE with not less than 5.5 per cent but not more than 10 per cent water; or CALCIUM HYPOCHLORITE, HYDRATED with not less than 5.5 per cent but not more than 10 per cent water | 5.1 |  | II |  | 1 |  | Forbidden | 5 |  |
| UN2881 | METAL CATALYST, DRY | 4.2 |  | I | 38 | 0 | 1 000 |  | Forbidden |  |
|  |  |  |  | II |  | 0 |  |  | Forbidden |  |
|  |  |  |  | III |  | 0 |  |  | 25 |  |
| UN2900 | INFECTIOUS SUBSTANCE, AFFECTING ANIMALS only | 6.2 |  | (RG 4) | 38 | 0 | 0 | 0.05 | 0.05 |  |
|  |  |  |  | (RG 3) | 38 | 0 |  | 0.05 | 0.05 |  |
|  |  |  |  | (RG 2) |  | 0 |  |  |  |  |
| UN2901 | BROMINE CHLORIDE | 2.3 (5.1) (8) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN2902 | PESTICIDE, LIQUID, TOXIC, N.O.S. | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN2903 | PESTICIDE, LIQUID, TOXIC, FLAMMABLE, N.O.S., flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN2904 | CHLOROPHENOLATES, LIQUID; or PHENOLATES, LIQUID | 8 |  | III |  | 5 |  |  | 5 | • |
| UN2905 | CHLOROPHENOLATES, SOLID; or PHENOLATES, SOLID | 8 |  | III |  | 5 |  |  | 25 | • |
| UN2907 | ISOSORBIDE DINITRATE MIXTURE with not less than 60 per cent lactose, mannose, starch or calcium hydrogen phosphate | 4.1 |  | II | 38 | 0 | 75 | Forbidden | 15 |  |
| UN2908 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - EMPTY PACKAGING | 7 |  |  | 72 | 0 |  |  | Forbidden |  |
| UN2909 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - ARTICLES MANUFACTURED FROM DEPLETED URANIUM; RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - ARTICLES MANUFACTURED FROM NATURAL THORIUM; or RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - ARTICLES MANUFACTURED FROM NATURAL URANIUM | 7 |  |  | 72 | 0 |  |  | Forbidden |  |
| UN2910 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - LIMITED QUANTITY OF MATERIAL | 7 |  |  | 72 | 0 |  |  |  |  |
| UN2911 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - ARTICLES; or RADIOACTIVE MATERIAL, EXCEPTED PACKAGE - INSTRUMENTS | 7 |  |  | 72 | 0 |  |  | Forbidden |  |
| UN2912 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-I) non-fissile or fissile excepted | 7 |  |  | 74 | 0 | 100 SOR/2002-306 |  |  |  |
| UN2913 | RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-I), non-fissile or fissile excepted; or RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-II), non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  |  |  |
| UN2915 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, non-special form, non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  | Forbidden |  |
| UN2916 | RADIOACTIVE MATERIAL, TYPE B(U) PACKAGE, non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  | Forbidden |  |
| UN2917 | RADIOACTIVE MATERIAL, TYPE B(M) PACKAGE, non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  | Forbidden |  |
| UN2919 | RADIOACTIVE MATERIAL, TRANSPORTED UNDER SPECIAL ARRANGEMENT, non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  | Forbidden |  |
| UN2920 | CORROSIVE LIQUID, FLAMMABLE, N.O.S. | 8 (3) |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 1 | • |
| UN2921 | CORROSIVE SOLID, FLAMMABLE, N.O.S. | 8 (4.1) |  | I | 16 | 0 | 3 000 |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
| UN2922 | CORROSIVE LIQUID, TOXIC, N.O.S. | 8 (6.1) |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 1 | • |
|  |  |  |  | III | 16 | 1 |  |  | 5 | • |
| UN2923 | CORROSIVE SOLID, TOXIC, N.O.S. | 8 (6.1) |  | I | 16 | 0 | 3 000, for Packing Group I only SOR/2002-306 |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 2 |  |  | 25 | • |
| UN2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. | 3 (8) |  | I | 16 | 0 | 1 000 | Forbidden | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN2925 | FLAMMABLE SOLID, CORROSIVE, ORGANIC, N.O.S. | 4.1 (8) |  | II | 16 | 0.5 | 1 000 | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 3 |  | Forbidden | 25 | • |
| UN2926 | FLAMMABLE SOLID, TOXIC, ORGANIC, N.O.S. | 4.1 (6.1) |  | II | 16 | 0.5 | 1 000 |  | 15 | • |
|  |  |  |  | III | 16 | 3 |  |  | 25 | • |
| UN2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. | 6.1 (8) |  | I | 16, 23 | 0 | 1 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 1 | • |
| UN2928 | TOXIC SOLID, CORROSIVE, ORGANIC, N.O.S. | 6.1 (8) |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 15 | • |
| UN2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
| UN2930 | TOXIC SOLID, FLAMMABLE, ORGANIC, N.O.S. | 6.1 (4.1) |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 15 | • |
| UN2931 | VANADYL SULFATE; or VANADYL SULPHATE | 6.1 |  | II |  | 0.5 |  |  | 25 |  |
| UN2933 | METHYL 2-CHLOROPROPIONATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2934 | ISOPROPYL 2-CHLOROPROPIONATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2935 | ETHYL 2-CHLOROPROPIONATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2936 | THIOLACTIC ACID | 6.1 |  | II |  | 0.5 |  |  | 5 |  |
| UN2937 | alpha-METHYLBENZYL ALCOHOL | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2940 | CYCLOOCTADIENE PHOSPHINES; or 9-PHOSPHABICYCLONONANES | 4.2 |  | II |  | 0 |  |  | 15 |  |
| UN2941 | FLUOROANILINES | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2942 | 2-TRIFLUOROMETHYLANILINE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2943 | TETRAHYDROFURFURYLAMINE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2945 | N-METHYLBUTYLAMINE | 3 (8) |  | II |  | 1 |  |  | 1 |  |
| UN2946 | 2-AMINO-5-DIETHYLAMINOPENTANE | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN2947 | ISOPROPYL CHLOROACETATE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN2948 | 3-TRIFLUOROMETHYLANILINE | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2949 | SODIUM HYDROSULFIDE with not less than 25 per cent water of crystallization; or SODIUM HYDROSULPHIDE with not less than 25 per cent water of crystallization | 8 |  | II |  | 1 |  |  | 15 |  |
| UN2950 | MAGNESIUM GRANULES, COATED, particle size not less than 149 microns | 4.3 |  | III |  | 1 |  |  | 25 |  |
| UN2956 | 5-tert-BUTYL-2,4,6-TRINITRO-m-XYLENE; or MUSK XYLENE | 4.1 |  | III | 38 | 3 | 75 | Forbidden | Forbidden | P |
| UN2965 | BORON TRIFLUORIDE DIMETHYL ETHERATE | 4.3 (3) (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2966 | THIOGLYCOL | 6.1 |  | II |  | 0.1 |  |  | 5 |  |
| UN2967 | SULFAMIC ACID; or SULPHAMIC ACID | 8 |  | III |  | 5 |  |  | 25 |  |
| UN2968 | MANEB PREPARATION, STABILIZED against self-heating; or MANEB, STABILIZED against self-heating | 4.3 |  | III |  | 1 |  |  | 25 | P |
| UN2969 | CASTOR BEANS; CASTOR FLAKE; CASTOR MEAL; or CASTOR POMACE | 9 |  | II |  | 5 |  | Forbidden |  |  |
| UN2977 | RADIOACTIVE MATERIAL, URANIUM HEXAFLUORIDE, FISSILE | 7 (8) |  |  |  | 0 | 25 |  |  |  |
| UN2978 | RADIOACTIVE MATERIAL, URANIUM HEXAFLUORIDE, non-fissile or fissile excepted | 7 (8) |  |  |  | 0 | 25 |  |  |  |
| UN2983 | ETHYLENE OXIDE AND PROPYLENE OXIDE MIXTURE, not more than 30 per cent ethylene oxide | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN2984 | HYDROGEN PEROXIDE, AQUEOUS SOLUTION with not less than 8 per cent but less than 20 per cent hydrogen peroxide (stabilized as necessary) | 5.1 |  | III |  | 5 |  |  | 2.5 |  |
| UN2985 | CHLOROSILANES, FLAMMABLE, CORROSIVE, N.O.S. | 3 (8) |  | II |  | 0 |  |  | 1 | • |
| UN2986 | CHLOROSILANES, CORROSIVE, FLAMMABLE, N.O.S. | 8 (3) |  | II |  | 0 |  |  | 1 | • |
| UN2987 | CHLOROSILANES, CORROSIVE, N.O.S. | 8 |  | II |  | 1 |  |  | 1 | • |
| UN2988 | CHLOROSILANES, WATER-REACTIVE, FLAMMABLE, CORROSIVE, N.O.S. | 4.3 (3) (8) |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN2989 | LEAD PHOSPHITE, DIBASIC | 4.1 |  | II |  | 1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 15 |  |
| UN2990 | LIFE-SAVING APPLIANCES, SELF-INFLATING | 9 |  |  | 21 | 0 |  |  |  |  |
| UN2991 | CARBAMATE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN2992 | CARBAMATE PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN2993 | ARSENICAL PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN2994 | ARSENICAL PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN2995 | ORGANOCHLORINE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN2996 | ORGANOCHLORINE PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN2997 | TRIAZINE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN2998 | TRIAZINE PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3005 | THIOCARBAMATE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN3006 | THIOCARBAMATE PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3009 | COPPER BASED PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN3010 | COPPER BASED PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3011 | MERCURY BASED PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | PP |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | PP |
|  |  |  |  | III | 16 | 1 |  |  | 60 | PP |
| UN3012 | MERCURY BASED PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | PP |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | PP |
|  |  |  |  | III | 16 | 5 |  |  | 60 | PP |
| UN3013 | SUBSTITUTED NITROPHENOL PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN3014 | SUBSTITUTED NITROPHENOL PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3015 | BIPYRIDILIUM PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN3016 | BIPYRIDILIUM PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3017 | ORGANOPHOSPHORUS PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN3018 | ORGANOPHOSPHORUS PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3019 | ORGANOTIN PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | PP |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | PP |
|  |  |  |  | III | 16 | 1 |  |  | 60 | PP |
| UN3020 | ORGANOTIN PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | PP |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | PP |
|  |  |  |  | III | 16 | 5 |  |  | 60 | PP |
| UN3021 | PESTICIDE, LIQUID, FLAMMABLE, TOXIC, N.O.S., flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN3022 | 1,2-BUTYLENE OXIDE, STABILIZED | 3 |  | II |  | 1 |  |  | 5 |  |
| UN3023 | 2-METHYL-2-HEPTANETHIOL | 6.1 (3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3024 | COUMARIN DERIVATIVE PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN3025 | COUMARIN DERIVATIVE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 60 | • |
| UN3026 | COUMARIN DERIVATIVE PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3027 | COUMARIN DERIVATIVE PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3028 | BATTERIES, DRY, CONTAINING POTASSIUM HYDROXIDE SOLID, electric storage | 8 |  | III |  | 5 |  |  | 25 |  |
| UN3048 | ALUMINUM PHOSPHIDE PESTICIDE | 6.1 |  | I | 38 | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3049 | METAL ALKYL HALIDES, WATER-REACTIVE, N.O.S.; or METAL ARYL HALIDES, WATER-REACTIVE, N.O.S. | 4.2 (4.3) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN3050 | METAL ALKYL HYDRIDES, WATER-REACTIVE, N.O.S.; or METAL ARYL HYDRIDES, WATER-REACTIVE, N.O.S. | 4.2 (4.3) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN3051 | ALUMINUM ALKYLS | 4.2 (4.3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3052 | ALUMINUM ALKYL HALIDES, LIQUID | 4.2 (4.3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3052 | ALUMINUM ALKYL HALIDES, SOLID | 4.2 (4.3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3053 | MAGNESIUM ALKYLS | 4.2 (4.3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3054 | CYCLOHEXYL MERCAPTAN | 3 |  | III |  | 5 |  |  | 60 |  |
| UN3055 | 2-(2-AMINOETHOXY)ETHANOL | 8 |  | III |  | 5 |  |  | 5 |  |
| UN3056 | n-HEPTALDEHYDE | 3 |  | III |  | 5 |  |  | 60 |  |
| UN3057 | TRIFLUOROACETYL CHLORIDE | 2.3 (8) |  |  |  | 0 | 25 | Forbidden | Forbidden |  |
| UN3064 | NITROGLYCERIN, SOLUTION IN ALCOHOL with more than 1 per cent but not more than 5 per cent nitroglycerin | 3 |  | II | 38 | 0 |  | Forbidden | Forbidden |  |
| UN3065 | ALCOHOLIC BEVERAGES, more than 70 per cent alcohol, by volume | 3 |  | II |  | 5 |  |  | 5 |  |
| UN3065 | ALCOHOLIC BEVERAGES, more than 24 per cent but not more than 70 per cent alcohol, by volume | 3 |  | III |  | 5 |  |  | 60 |  |
| UN3066 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer base) with not more than 20 per cent nitrocellulose by mass if the nitrogen content of the nitrocellulose is not more than 12.6 per cent by mass; or PAINT RELATED MATERIAL (including paint thinning or reducing compound) with not more than 20 per cent nitrocellulose by mass if the nitrogen content of the nitrocellulose is not more than 12.6 per cent by mass | 8 |  | II | 59 | 1 |  |  | 1 | • |
|  |  |  |  | III | 59 | 5 |  |  | 5 | • |
| UN3070 | ETHYLENE OXIDE AND DICHLORODIFLUOROMETHANE MIXTURE with not more than 12.5 per cent ethylene oxide | 2.2 |  |  |  | 0.12 |  |  | 75 |  |
| UN3071 | MERCAPTAN MIXTURE, LIQUID, TOXIC, FLAMMABLE, N.O.S.; or MERCAPTANS, LIQUID, TOXIC, FLAMMABLE, N.O.S. | 6.1 (3) |  | II | 16 | 0.1 | 1 000 |  | 5 | • |
| UN3072 | LIFE-SAVING APPLIANCES NOT SELF-INFLATING, containing dangerous goods as equipment | 9 |  |  | 21 | 0 |  |  |  |  |
| UN3073 | VINYLPYRIDINES, STABILIZED | 6.1 (3) (8) |  | II |  | 0.1 | 1 000 |  | 1 |  |
| UN3076 | ALUMINUM ALKYL HYDRIDES | 4.2 (4.3) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3077 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. | 9 |  | III | 16 | 5 |  |  |  | • |
| UN3078 | CERIUM, turnings or gritty powder | 4.3 |  | II |  | 0.5 |  | Forbidden | 15 |  |
| UN3079 | METHACRYLONITRILE, STABILIZED | 3 (6.1) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3080 | ISOCYANATE SOLUTION, TOXIC, FLAMMABLE, N.O.S.; or ISOCYANATES, TOXIC, FLAMMABLE, N.O.S. | 6.1 (3) |  | II | 16, 23 | 0.1 | 1 000 | Forbidden | 5 | • |
| UN3082 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. | 9 |  | III | 16 | 5 |  |  |  | • |
| UN3083 | PERCHLORYL FLUORIDE | 2.3 (5.1) |  |  | 38 | 0 | 25 | Forbidden | Forbidden |  |
| UN3084 | CORROSIVE SOLID, OXIDIZING, N.O.S. | 8 (5.1) |  | I | 16 | 0 |  |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
| UN3085 | OXIDIZING SOLID, CORROSIVE, N.O.S. | 5.1 (8) |  | I | 16 | 0 | 1 000 | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 25 | • |
| UN3086 | TOXIC SOLID, OXIDIZING, N.O.S. | 6.1 (5.1) |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 5 | • |
| UN3087 | OXIDIZING SOLID, TOXIC, N.O.S. | 5.1 (6.1) |  | I | 16 | 0 | 1 000 | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 5 | • |
|  |  |  |  | III | 16 | 1 |  |  | 25 | • |
| UN3088 | SELF-HEATING SOLID, ORGANIC, N.O.S. | 4.2 |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3089 | METAL POWDER, FLAMMABLE, N.O.S. | 4.1 |  | II |  | 1 |  |  | 15 | • |
|  |  |  |  | III |  | 5 |  |  | 25 | • |
| UN3090 | LITHIUM BATTERIES | 9 |  | II | 34 | 0 |  |  | 5 |  |
| UN3091 | LITHIUM BATTERIES CONTAINED IN EQUIPMENT; or LITHIUM BATTERIES PACKED WITH EQUIPMENT | 9 |  | II | 34 | 0 |  |  | 5 |  |
| UN3092 | 1-METHOXY-2-PROPANOL | 3 |  | III |  | 5 |  |  | 60 |  |
| UN3093 | CORROSIVE LIQUID, OXIDIZING, N.O.S. | 8 (5.1) |  | I | 16 | 0 | 3 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 1 | • |
| UN3094 | CORROSIVE LIQUID, WATER-REACTIVE, N.O.S. | 8 (4.3) |  | I | 16, 38 | 0 | 3 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 1 | • |
| UN3095 | CORROSIVE SOLID, SELF-HEATING, N.O.S. | 8 (4.2) |  | I | 16, 38 | 0 |  | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 1 |  | Forbidden | 15 | • |
| UN3096 | CORROSIVE SOLID, WATER-REACTIVE, N.O.S. | 8 (4.3) |  | I | 16, 38 | 0 |  | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 1 |  | Forbidden | 15 | • |
| UN3097 | FLAMMABLE SOLID, OXIDIZING, N.O.S. | Forbidden |  |  |  |  |  |  |  |  |
| UN3098 | OXIDIZING LIQUID, CORROSIVE, N.O.S. | 5.1 (8) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 1 | • |
|  |  |  |  | III | 16 | 1 |  |  | 2.5 | • |
| UN3099 | OXIDIZING LIQUID, TOXIC, N.O.S. | 5.1 (6.1) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 1 | • |
|  |  |  |  | III | 16 | 1 |  |  | 2.5 | • |
| UN3100 | OXIDIZING SOLID, SELF-HEATING, N.O.S. | Forbidden |  |  |  |  |  |  |  |  |
| UN3101 | ORGANIC PEROXIDE TYPE B, LIQUID | 5.2 (1) |  | II | 16, 38 | 0.025 | 75 | Forbidden | Forbidden |  |
| UN3102 | ORGANIC PEROXIDE TYPE B, SOLID | 5.2 (1) |  | II | 16, 38 | 0.1 | 75 | Forbidden | Forbidden |  |
| UN3103 | ORGANIC PEROXIDE TYPE C, LIQUID | 5.2 |  | II | 16, 38 | 0.025 |  | Forbidden | 5 |  |
| UN3104 | ORGANIC PEROXIDE TYPE C, SOLID | 5.2 |  | II | 16, 38 | 0.1 |  | Forbidden | 5 |  |
| UN3105 | ORGANIC PEROXIDE TYPE D, LIQUID | 5.2 |  | II | 16, 38 | 0.125 |  | Forbidden | 5 |  |
| UN3106 | ORGANIC PEROXIDE TYPE D, SOLID | 5.2 |  | II | 16, 38 | 0.5 |  | Forbidden | 5 |  |
| UN3107 | ORGANIC PEROXIDE TYPE E, LIQUID | 5.2 |  | II | 16, 38 | 0.125 |  | Forbidden | 10 |  |
| UN3108 | ORGANIC PEROXIDE TYPE E, SOLID | 5.2 |  | II | 16, 38 | 0.5 |  | Forbidden | 10 |  |
| UN3109 | ORGANIC PEROXIDE TYPE F, LIQUID | 5.2 |  | II | 16 | 0.125 |  | Forbidden | 10 |  |
| UN3110 | ORGANIC PEROXIDE TYPE F, SOLID | 5.2 |  | II | 16 | 0.5 |  | Forbidden | 10 |  |
| UN3111 | ORGANIC PEROXIDE TYPE B, LIQUID, TEMPERATURE CONTROLLED | 5.2 (1) |  | II | 16, 28 | 0 | 75 | Forbidden | Forbidden |  |
| UN3112 | ORGANIC PEROXIDE TYPE B, SOLID, TEMPERATURE CONTROLLED | 5.2 (1) |  | II | 16, 28 | 0 | 75 | Forbidden | Forbidden | • |
| UN3113 | ORGANIC PEROXIDE TYPE C, LIQUID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16, 28 | 0 | 75 | Forbidden | Forbidden |  |
| UN3114 | ORGANIC PEROXIDE TYPE C, SOLID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16, 28 | 0 | 75 | Forbidden | Forbidden |  |
| UN3115 | ORGANIC PEROXIDE TYPE D, LIQUID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16, 28 | 0 |  | Forbidden | Forbidden |  |
| UN3116 | ORGANIC PEROXIDE TYPE D, SOLID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16, 28 | 0 |  | Forbidden | Forbidden |  |
| UN3117 | ORGANIC PEROXIDE TYPE E, LIQUID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16, 28 | 0 |  | Forbidden | Forbidden |  |
| UN3118 | ORGANIC PEROXIDE TYPE E, SOLID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16, 28 | 0 |  | Forbidden | Forbidden |  |
| UN3119 | ORGANIC PEROXIDE TYPE F, LIQUID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16 | 0 |  | Forbidden | Forbidden |  |
| UN3120 | ORGANIC PEROXIDE TYPE F, SOLID, TEMPERATURE CONTROLLED | 5.2 |  | II | 16 | 0 | SOR/2002-306 | Forbidden | Forbidden |  |
| UN3121 | OXIDIZING SOLID, WATER-REACTIVE, N.O.S. | Forbidden |  |  |  |  |  |  |  |  |
| UN3122 | TOXIC LIQUID, OXIDIZING, N.O.S. | 6.1 (5.1) |  | I | 16, 23 | 0 | 3 000 |  | Forbidden | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 1 | • |
| UN3123 | TOXIC LIQUID, WATER-REACTIVE, N.O.S. | 6.1 (4.3) |  | I | 16, 23 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.1 | 3 000 | Forbidden | 1 | • |
| UN3124 | TOXIC SOLID, SELF-HEATING, N.O.S. | 6.1 |  | I | 16, 38 | 0 | 1 000 | Forbidden | 5 | • |
|  |  | (4.2) |  |  |  |  |  |  |  |  |
|  |  |  |  | II | 16 | 0 |  | Forbidden | 15 | • |
| UN3125 | TOXIC SOLID, WATER-REACTIVE, N.O.S. | 6.1 (4.3) |  | I | 16, 38 | 0 | 1 000 | Forbidden | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 15 | • |
| UN3126 | SELF-HEATING SOLID, CORROSIVE, ORGANIC, N.O.S. | 4.2 (8) |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3127 | SELF-HEATING SOLID, OXIDIZING, N.O.S. | Forbidden |  |  |  |  |  |  |  |  |
| UN3128 | SELF-HEATING SOLID, TOXIC, ORGANIC, N.O.S. | 4.2 (6.1) |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3129 | WATER-REACTIVE LIQUID, CORROSIVE, N.O.S. | 4.3 (8) |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 1 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 5 | • |
| UN3130 | WATER-REACTIVE LIQUID, TOXIC, N.O.S. | 4.3 (6.1) |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 1 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 5 | • |
| UN3131 | WATER-REACTIVE SOLID, CORROSIVE, N.O.S. | 4.3 (8) |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 25 | • |
| UN3132 | WATER-REACTIVE SOLID, FLAMMABLE, N.O.S. | 4.3 (4.1) |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden |  |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 15 |  |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 25 |  |
| UN3133 | WATER-REACTIVE SOLID, OXIDIZING, N.O.S. | Forbidden |  |  |  |  |  |  |  |  |
| UN3134 | WATER-REACTIVE SOLID, TOXIC, N.O.S. | 4.3 (6.1) |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 25 | • |
| UN3135 | WATER-REACTIVE SOLID, SELF-HEATING, N.O.S. | 4.3 (4.2) |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3136 | TRIFLUOROMETHANE, REFRIGERATED LIQUID | 2.2 |  |  |  | 0.125 SOR/2002-306 |  | Forbidden | 50 |  |
| UN3137 | OXIDIZING SOLID, FLAMMABLE, N.O.S. | Forbidden |  |  |  |  |  |  |  |  |
| UN3138 | ETHYLENE, ACETYLENE AND PROPYLENE MIXTURE, REFRIGERATED LIQUID containing at least 71.5 percent ethylene with not more than 22.5 percent acetylene and not more than 6 percent propylene | 2.1 |  |  |  | 0.125 SOR/2002-306 |  | Forbidden | Forbidden |  |
| UN3139 | OXIDIZING LIQUID, N.O.S. | 5.1 |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 2.5 | • |
| UN3140 | ALKALOID SALTS, LIQUID, N.O.S; or ALKALOIDS, LIQUID, N.O.S. | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3141 | ANTIMONY COMPOUND, INORGANIC, LIQUID, N.O.S. except antimony oxides and sulphides containing less than 0.5 per cent arsenic, by mass | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN3142 | DISINFECTANT, LIQUID, TOXIC, N.O.S. | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3143 | DYE INTERMEDIATE, SOLID, TOXIC, N.O.S.; or DYE, SOLID, TOXIC, N.O.S. | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3144 | NICOTINE COMPOUND, LIQUID, N.O.S.; or NICOTINE PREPARATION, LIQUID, N.O.S. | 6.1 |  | I | 23 | 0 | 1 000 |  | 1 |  |
|  |  |  |  | II |  | 0.1 |  |  | 5 |  |
|  |  |  |  | III |  | 5 |  |  | 60 |  |
| UN3145 | ALKYLPHENOLS, LIQUID, N.O.S. including C2-C12 homologues | 8 |  | I | 50 | 0 | 3 000, for Packing Group I only SOR/2002-306 |  | 0.5 | • |
|  |  |  |  | II | 50 | 1 |  |  | 1 | • |
|  |  |  |  | III | 50 | 5 |  |  | 5 | • |
| UN3146 | ORGANOTIN COMPOUND, SOLID, N.O.S. | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | PP |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | PP |
|  |  |  |  | III | 16 | 5 |  |  | 100 | PP |
| UN3147 | DYE INTERMEDIATE, SOLID, CORROSIVE, N.O.S.; or DYE, SOLID, CORROSIVE, N.O.S. | 8 |  | I | 16 | 0 | 3 000, for Packing Group I only SOR/2002-306 |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3148 | WATER-REACTIVE LIQUID, N.O.S. | 4.3 |  | I | 16, 38 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 1 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 5 | • |
| UN3149 | HYDROGEN PEROXIDE AND PEROXYACETIC ACID MIXTURE STABILIZED with acid(s), water and not more than 5 per cent peroxyacetic acid | 5.1 (8) |  | II |  | 0.5 |  | Forbidden | 1 |  |
| UN3150 | DEVICES, SMALL, HYDROCARBON GAS POWERED with release device; or HYDROCARBON GAS REFILLS FOR SMALL DEVICES with release device | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 |  | 1 |  |
| UN3151 | POLYHALOGENATED BIPHENYLS, LIQUID regulated only when the concentration is more than 50 ppm by mass; or POLYHALOGENATED TERPHENYLS, LIQUID regulated only when the concentration is more than 50 ppm by mass | 9 |  | II |  | 1 |  |  | 100 | PP |
| UN3152 | POLYHALOGENATED BIPHENYLS, SOLID regulated only when the concentration is more than 50 ppm by mass; or POLYHALOGENATED TERPHENYLS, SOLID regulated only when the concentration is more than 50 ppm by mass | 9 |  | II |  | 1 |  |  | 100 | PP |
| UN3153 | PERFLUORO(METHYLVINYL ETHER) | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN3154 | PERFLUORO(ETHYLVINYL ETHER) | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN3155 | PENTACHLOROPHENOL | 6.1 |  | II |  | 0.5 |  |  | 25 | PP |
| UN3156 | COMPRESSED GAS, OXIDIZING, N.O.S. | 2.2 (5.1) |  |  | 16 | 0 | 3 000 | Forbidden | 75 | • |
| UN3157 | LIQUEFIED GAS, OXIDIZING, N.O.S. | 2.2 (5.1) |  |  | 16 | 0 | 3 000 | Forbidden | 75 | • |
| UN3158 | GAS, REFRIGERATED LIQUID, N.O.S. | 2.2 |  |  | 16 | 0.125 SOR/2002-306 |  | Forbidden | 50 | • |
| UN3159 | REFRIGERANT GAS R 134a; or 1,1,1,2-TETRAFLUOROETHANE | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3160 | LIQUEFIED GAS, TOXIC, FLAMMABLE, N.O.S. | 2.3 (2.1) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3161 | LIQUEFIED GAS, FLAMMABLE, N.O.S. | 2.1 |  |  | 16 | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden | • |
| UN3162 | LIQUEFIED GAS, TOXIC, N.O.S. | 2.3 |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3163 | LIQUEFIED GAS, N.O.S. | 2.2 |  |  | 16 | 0.125 SOR/2002-306 |  |  | 75 | • |
| UN3164 | ARTICLES, PRESSURIZED, HYDRAULIC (containing non-flammable gas); or ARTICLES, PRESSURIZED, PNEUMATIC (containing non-flammable gas) | 2.2 |  |  | 40 | 0.125 SOR/2002-306 |  |  |  | • |
| UN3165 | AIRCRAFT HYDRAULIC POWER UNIT FUEL TANK containing a mixture of anhydrous hydrazine and methylhydrazine (M86 fuel) | 3 (6.1) (8) |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3166 | ENGINES, INTERNAL COMBUSTION (FLAMMABLE GAS POWERED) including when fitted in machinery or vehicles, regulated by aircraft only (ICAO terminology) | 9 |  |  | 63 | 0 |  |  |  |  |
| UN3166 | ENGINES, INTERNAL COMBUSTION (FLAMMABLE LIQUID POWERED) including when fitted in machinery or vehicles, regulated by aircraft only (ICAO terminology) | 9 |  |  | 63 | 0 |  |  |  |  |
| UN3167 | GAS SAMPLE, NON-PRESSURIZED, FLAMMABLE, N.O.S., not refrigerated liquid | 2.1 |  |  |  | 0.125 SOR/2002-306 |  | Forbidden | 1 | • |
| UN3168 | GAS SAMPLE, NON-PRESSURIZED, TOXIC, FLAMMABLE, N.O.S., not refrigerated liquid | 2.3 (2.1) |  |  |  | 0 |  | Forbidden | Forbidden | • |
| UN3169 | GAS SAMPLE, NON-PRESSURIZED, TOXIC, N.O.S., not refrigerated liquid | 2.3 |  |  |  | 0 |  | Forbidden | Forbidden | • |
| UN3170 | ALUMINUM REMELTING BY-PRODUCTS including, but not limited to, aluminum dross, aluminum skimmings, spent cathodes, spent potliner and aluminum salt slags; or ALUMINUM SMELTING BY-PRODUCTS including, but not limited to, aluminum dross, aluminum skimmings, spent cathodes, spent potliner and aluminum salt slags | 4.3 |  | II |  | 0.5 |  |  | 15 |  |
|  |  |  |  | III |  | 1 |  |  | 25 |  |
| UN3171 | BATTERY-POWERED EQUIPMENT, regulated by aircraft only; or BATTERY-POWERED VEHICLE, regulated by aircraft only | 9 |  |  | 63, 67 | 0 |  |  |  |  |
| UN3172 | TOXINS, EXTRACTED FROM LIVING SOURCES, LIQUID, N.O.S., not containing infectious substances | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3172 | TOXINS, EXTRACTED FROM LIVING SOURCES, SOLID, N.O.S., not containing infectious substances | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3174 | TITANIUM DISULFIDE; or TITANIUM DISULPHIDE | 4.2 |  | III |  | 0 |  |  | 25 |  |
| UN3175 | SOLIDS CONTAINING FLAMMABLE LIQUID, N.O.S. | 4.1 |  | II | 16, 56 | 1 |  |  | 15 | • |
| UN3176 | FLAMMABLE SOLID, ORGANIC, MOLTEN, N.O.S. | 4.1 |  | II | 16 | 0 |  |  | Forbidden | • |
|  |  |  |  | III | 16 | 0 |  |  |  | • |
| UN3178 | FLAMMABLE SOLID, INORGANIC, N.O.S. | 4.1 |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3179 | FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. | 4.1 (6.1) |  | II | 16 | 0.5 |  |  | 15 | • |
|  |  |  |  | III | 16 | 3 |  |  | 25 | • |
| UN3180 | FLAMMABLE SOLID, CORROSIVE, INORGANIC, N.O.S. | 4.1 (8) |  | II | 16 | 0.5 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 3 |  | Forbidden | 25 | • |
| UN3181 | METAL SALTS OF ORGANIC COMPOUNDS, FLAMMABLE, N.O.S. | 4.1 |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3182 | METAL HYDRIDES, FLAMMABLE, N.O.S. | 4.1 |  | II | 16 | 1 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 5 |  | Forbidden | 25 | • |
| UN3183 | SELF-HEATING LIQUID, ORGANIC, N.O.S. | 4.2 |  | II | 16 | 0 |  |  | 1 | • |
|  |  |  |  | III | 16 | 0 |  |  | 5 | • |
| UN3184 | SELF-HEATING LIQUID, TOXIC, ORGANIC, N.O.S. | 4.2 (6.1) |  | II | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | III | 16 | 0 |  |  | 5 | • |
| UN3185 | SELF-HEATING LIQUID, CORROSIVE, ORGANIC, N.O.S. | 4.2 (8) |  | II | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | III | 16 | 0 |  |  | 5 | • |
| UN3186 | SELF-HEATING LIQUID, INORGANIC, N.O.S. | 4.2 |  | II | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | III | 16 | 0 |  |  | 5 | • |
| UN3187 | SELF-HEATING LIQUID, TOXIC, INORGANIC, N.O.S. | 4.2 (6.1) |  | II | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | III | 16 | 0 |  |  | 5 | • |
| UN3188 | SELF-HEATING LIQUID, CORROSIVE, INORGANIC, N.O.S. | 4.2 (8) |  | II | 16 | 0 |  |  | 1 | • |
|  |  |  |  | III | 16 | 0 |  |  | 5 | • |
| UN3189 | METAL POWDER, SELF-HEATING, N.O.S. | 4.2 |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3190 | SELF-HEATING SOLID, INORGANIC, N.O.S. | 4.2 |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3191 | SELF-HEATING SOLID, TOXIC, INORGANIC, N.O.S. | 4.2 (6.1) |  | II | 16 | 0 | 1 000 |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3192 | SELF-HEATING SOLID, CORROSIVE, INORGANIC, N.O.S. | 4.2 (8) |  | II | 16 | 0 | 1 000 |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3194 | PYROPHORIC LIQUID, INORGANIC, N.O.S. | 4.2 |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN3200 | PYROPHORIC SOLID, INORGANIC, N.O.S. | 4.2 |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN3203 | PYROPHORIC ORGANOMETALLIC COMPOUND, WATER-REACTIVE, N.O.S., liquid | 4.2 (4.3) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN3203 | PYROPHORIC ORGANOMETALLIC COMPOUND, WATER-REACTIVE, N.O.S., solid | 4.2 (4.3) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
| UN3205 | ALKALINE EARTH METAL ALCOHOLATES, N.O.S. | 4.2 |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3206 | ALKALI METAL ALCOHOLATES, SELF-HEATING, CORROSIVE, N.O.S. | 4.2 (8) |  | II | 16 | 0 |  |  | 15 | • |
|  |  |  |  | III | 16 | 0 |  |  | 25 | • |
| UN3207 | ORGANOMETALLIC COMPOUND DISPERSION, WATER-REACTIVE, FLAMMABLE, N.O.S.; ORGANOMETALLIC COMPOUND SOLUTION, WATER-REACTIVE, FLAMMABLE, N.O.S.; or ORGANOMETALLIC COMPOUND, WATER-REACTIVE, FLAMMABLE, N.O.S. | 4.3 (3) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 1 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 5 | • |
| UN3208 | METALLIC SUBSTANCE, WATER-REACTIVE, N.O.S. | 4.3 |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0.5 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 1 |  | Forbidden | 25 | • |
| UN3209 | METALLIC SUBSTANCE, WATER-REACTIVE, SELF-HEATING, N.O.S. | 4.3 (4.2) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 0 |  | Forbidden | 15 | • |
|  |  |  |  | III | 16 | 0 |  | Forbidden | 25 | • |
| UN3210 | CHLORATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | II | 68 | 1 |  |  | 1 | • |
|  |  |  |  | III |  | 5 |  |  | 2.5 | • |
| UN3211 | PERCHLORATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | II |  | 1 |  |  | 1 | • |
|  |  |  |  | III |  | 5 |  |  | 2.5 | • |
| UN3212 | HYPOCHLORITES, INORGANIC, N.O.S. | 5.1 |  | II | 68 | 1 |  | Forbidden | 5 | • |
| UN3213 | BROMATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | II | 68 | 1 |  |  | 1 | • |
|  |  |  |  | III |  | 5 |  |  | 2.5 | • |
| UN3214 | PERMANGANATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | II | 68 | 1 |  | Forbidden | 1 | • |
| UN3215 | PERSULFATES, INORGANIC, N.O.S.; or PERSULPHATES, INORGANIC, N.O.S. | 5.1 |  | III |  | 5 |  |  | 25 | • |
| UN3216 | PERSULFATES, INORGANIC, AQUEOUS SOLUTION, N.O.S.; or PERSULPHATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | III |  | 5 |  |  | 2.5 | • |
| UN3218 | NITRATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | II | 55 | 1 |  |  | 1 | • |
|  |  |  |  | III | 55 | 5 |  |  | 2.5 | • |
| UN3219 | NITRITES, INORGANIC, AQUEOUS SOLUTION, N.O.S. | 5.1 |  | II | 68 | 1 |  |  | 1 | • |
|  |  |  |  | III |  | 5 |  |  | 2.5 | • |
| UN3220 | PENTAFLUOROETHANE; or REFRIGERANT GAS R 125 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3221 | SELF-REACTIVE LIQUID TYPE B | 4.1 |  | II | 16, 38 | 0 | 75 | Forbidden | Forbidden |  |
|  |  | (1) |  |  |  |  |  |  |  |  |
| UN3222 | SELF-REACTIVE SOLID TYPE B | 4.1 |  | II | 16, 38 | 0 | 75 | Forbidden | Forbidden |  |
|  |  | (1) |  |  |  |  |  |  |  |  |
| UN3223 | SELF-REACTIVE LIQUID TYPE C | 4.1 |  | II | 16, 38 | 1 | 3 000 | Forbidden | 5 |  |
| UN3224 | SELF-REACTIVE SOLID TYPE C | 4.1 |  | II | 16, 38 | 1 | 3 000 | Forbidden | 5 |  |
| UN3225 | SELF-REACTIVE LIQUID TYPE D | 4.1 |  | II | 16, 38 | 1 |  | Forbidden | 5 |  |
| UN3226 | SELF-REACTIVE SOLID TYPE D | 4.1 |  | II | 16, 38 | 1 |  | Forbidden | 5 |  |
| UN3227 | SELF-REACTIVE LIQUID TYPE E | 4.1 |  | II | 16, 38 | 1 |  | Forbidden | 10 |  |
| UN3228 | SELF-REACTIVE SOLID TYPE E | 4.1 |  | II | 16, 38 | 1 |  | Forbidden | 10 |  |
| UN3229 | SELF-REACTIVE LIQUID TYPE F | 4.1 |  | II | 16, 38 | 1 |  | Forbidden | 10 |  |
| UN3230 | SELF-REACTIVE SOLID TYPE F | 4.1 |  | II | 16, 38 | 1 |  | Forbidden | 10 |  |
| UN3231 | SELF-REACTIVE LIQUID TYPE B, TEMPERATURE CONTROLLED | 4.1 (1) |  | II | 16, 28 | 0 | 75 | Forbidden | Forbidden |  |
| UN3232 | SELF-REACTIVE SOLID TYPE B, TEMPERATURE CONTROLLED | 4.1 (1) |  | II | 16, 28 | 0 | 75 | Forbidden | Forbidden |  |
| UN3233 | SELF-REACTIVE LIQUID TYPE C, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 | 75 | Forbidden | Forbidden |  |
| UN3234 | SELF-REACTIVE SOLID TYPE C, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 | 75 | Forbidden | Forbidden |  |
| UN3235 | SELF-REACTIVE LIQUID TYPE D, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 | 75 |  | Forbidden |  |
| UN3236 | SELF-REACTIVE SOLID TYPE D, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 |  | Forbidden | Forbidden |  |
| UN3237 | SELF-REACTIVE LIQUID TYPE E, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 |  | Forbidden | Forbidden |  |
| UN3238 | SELF-REACTIVE SOLID TYPE E, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 |  | Forbidden | Forbidden |  |
| UN3239 | SELF-REACTIVE LIQUID TYPE F, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 |  | Forbidden | Forbidden |  |
| UN3240 | SELF-REACTIVE SOLID TYPE F, TEMPERATURE CONTROLLED | 4.1 |  | II | 16, 28 | 1 |  | Forbidden | Forbidden |  |
| UN3241 | 2-BROMO-2-NITROPROPANE-1,3-DIOL | 4.1 |  | III | 73 | 5 |  |  | 25 |  |
| UN3242 | AZODICARBONAMIDE, technically pure or preparations having an SADT higher than 75 °C | 4.1 |  | II |  | 1 |  | Forbidden | Forbidden | P |
| UN3243 | SOLIDS CONTAINING TOXIC LIQUID, N.O.S. | 6.1 |  | II | 16, 57 | 0.5 |  |  | 25 | • |
| UN3244 | SOLIDS CONTAINING CORROSIVE LIQUID, N.O.S. | 8 |  | II | 16, 26, 58 | 1 |  |  | 15 | • |
| UN3245 | GENETICALLY MODIFIED MICRO-ORGANISMS not containing infectious substances | 9 |  |  |  | 0 |  | Forbidden |  |  |
| UN3246 | METHANESULFONYL CHLORIDE; or METHANESULPHONYL CHLORIDE | 6.1 (8) |  | I |  | 0 | 1 000 | Forbidden | 0.5 |  |
| UN3247 | SODIUM PEROXOBORATE, ANHYDROUS | 5.1 |  | II |  | 1 |  |  | 5 |  |
| UN3248 | MEDICINE, LIQUID, FLAMMABLE, TOXIC, N.O.S. | 3 (6.1) |  | II | 16, 38 | 1 | 1 000 |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN3249 | MEDICINE, SOLID, TOXIC, N.O.S. | 6.1 |  | II | 16, 38 | 0.5 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN3250 | CHLOROACETIC ACID, MOLTEN | 6.1 (8) |  | II |  | 0 | 1 000 |  | Forbidden |  |
| UN3251 | ISOSORBIDE-5-MONONITRATE with less than 30 per cent non-volatile, non-flammable phlegmatizer | 4.1 |  | III |  | 5 |  | Forbidden | Forbidden |  |
| UN3252 | DIFLUOROMETHANE; or REFRIGERANT GAS R 32 | 2.1 |  |  |  | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden |  |
| UN3253 | DISODIUM TRIOXOSILICATE | 8 |  | III |  | 5 |  |  | 25 |  |
| UN3254 | TRIBUTYLPHOSPHANE | 4.2 |  | I |  | 0 | 1 000 | Forbidden | Forbidden |  |
| UN3255 | tert-BUTYL HYPOCHLORITE | Forbidden |  |  |  |  |  |  |  |  |
| UN3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S., with flash point above 60.5 °C, at or above its flash point | 3 |  | III |  | 0 |  |  | Forbidden | • |
| UN3257 | ELEVATED TEMPERATURE LIQUID, N.O.S., at or above 100 °C and below its flash point including molten metals, molten salts, etc. | 9 |  | III |  | 5 |  |  | Forbidden | • |
| UN3258 | ELEVATED TEMPERATURE SOLID, N.O.S., at or above 240 °C | 9 |  | III |  | 5 |  |  | Forbidden | • |
| UN3259 | AMINES, SOLID, CORROSIVE, N.O.S.; or POLYAMINES, SOLID, CORROSIVE, N.O.S. | 8 |  | I | 16 | 0 | 3 000 |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3260 | CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. | 8 |  | I | 16 | 0 |  |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3261 | CORROSIVE SOLID, ACIDIC, ORGANIC, N.O.S. | 8 |  | I | 16 | 0 |  |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3262 | CORROSIVE SOLID, BASIC, INORGANIC, N.O.S. | 8 |  | I | 16 | 0 |  |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3263 | CORROSIVE SOLID, BASIC, ORGANIC, N.O.S. | 8 |  | I | 16 | 0 |  |  | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 15 | • |
|  |  |  |  | III | 16 | 5 |  |  | 25 | • |
| UN3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. | 8 |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN3265 | CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. | 8 |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN3266 | CORROSIVE LIQUID, BASIC, INORGANIC, N.O.S. | 8 |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN3267 | CORROSIVE LIQUID, BASIC, ORGANIC, N.O.S. | 8 |  | I | 16 | 0 | 3 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
|  |  |  |  | III | 16 | 5 |  |  | 5 | • |
| UN3268 | AIR BAG INFLATORS, pyrotechnic; AIR BAG MODULES, pyrotechnic; or SEAT-BELT PRETENSIONERS, pyrotechnic | 9 |  | III | 25 | 5 |  |  | 25 |  |
| UN3269 | POLYESTER RESIN KIT | 3 |  | II |  | 5 |  |  | 5 | • |
|  |  |  |  | III |  | 5 |  |  | 5 | • |
| UN3270 | NITROCELLULOSE MEMBRANE FILTERS, with not more than 12.6 percent nitrogen, by dry mass | 4.1 |  | II |  | 0 |  | Forbidden | 1 |  |
| UN3271 | ETHERS, N.O.S. | 3 |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3272 | ESTERS, N.O.S. | 3 |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3273 | NITRILES, FLAMMABLE, TOXIC, N.O.S. | 3 (6.1) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN3274 | ALCOHOLATES SOLUTION, N.O.S., in alcohol | 3 (8) |  | II | 16 | 1 |  |  | 1 | • |
| UN3275 | NITRILES, TOXIC, FLAMMABLE, N.O.S. | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
| UN3276 | NITRILES, TOXIC, liquid, N.O.S.; or NITRILES, TOXIC, LIQUID, N.O.S. (ICAO terminology) | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3276 | NITRILES, TOXIC, solid, N.O.S.; or NITRILES, TOXIC, SOLID, N.O.S. (ICAO terminology) | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3277 | CHLOROFORMATES, TOXIC, CORROSIVE, N.O.S. | 6.1 (8) |  | II | 16 | 0.1 | 1 000 |  | 1 | • |
| UN3278 | ORGANOPHOSPHORUS COMPOUND, TOXIC, N.O.S., liquid | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3278 | ORGANOPHOSPHORUS COMPOUND, TOXIC, N.O.S., solid | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3279 | ORGANOPHOSPHORUS COMPOUND, TOXIC, FLAMMABLE, N.O.S. | 6.1 (3) |  | I | 16, 23 | 0 | 1 000 |  | 1 |  |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 |  |
| UN3280 | ORGANOARSENIC COMPOUND, N.O.S., liquid | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3280 | ORGANOARSENIC COMPOUND, N.O.S., solid | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3281 | METAL CARBONYLS, N.O.S., liquid | 6.1 |  | I | 16 | 0 | 1 000 | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3281 | METAL CARBONYLS, N.O.S., solid | 6.1 |  | I | 16, 23 | 0 | 1 000 | Forbidden | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3282 | ORGANOMETALLIC COMPOUND, TOXIC, N.O.S., liquid | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3282 | ORGANOMETALLIC COMPOUND, TOXIC, N.O.S., solid | 6.1 |  | I | 16, 23 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3283 | SELENIUM COMPOUND, N.O.S. | 6.1 |  | I |  | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II |  | 0.5 |  |  | 25 | • |
|  |  |  |  | III |  | 5 |  |  | 100 | • |
| UN3284 | TELLURIUM COMPOUND, N.O.S. | 6.1 |  | I |  | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II |  | 0.5 |  |  | 25 | • |
|  |  |  |  | III |  | 5 |  |  | 100 | • |
| UN3285 | VANADIUM COMPOUND, N.O.S. | 6.1 |  | I |  | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II |  | 0.5 |  |  | 25 | • |
|  |  |  |  | III |  | 5 |  |  | 100 | • |
| UN3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. | 3 (6.1) (8) |  | I | 16 | 0 | 1 000 | Forbidden | Forbidden | • |
|  |  |  |  | II | 16 | 1 |  |  | 1 | • |
| UN3287 | TOXIC LIQUID, INORGANIC, N.O.S. | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3288 | TOXIC SOLID, INORGANIC, N.O.S. | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 | • |
|  |  |  |  | III | 16 | 5 |  |  | 100 | • |
| UN3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. | 6.1 (8) |  | I | 16 | 0 | 1 000 |  | 0.5 | • |
|  |  |  |  | II | 16 | 0.1 |  |  | 1 | • |
| UN3290 | TOXIC SOLID, CORROSIVE, INORGANIC, N.O.S. | 6.1 (8) |  | I | 16 | 0 | 1 000 |  | 1 | • |
|  |  |  |  | II | 16 | 0.5 |  |  | 15 | • |
| UN3292 | BATTERIES, CONTAINING SODIUM; or CELLS, CONTAINING SODIUM | 4.3 |  | II |  | 0 |  |  | Forbidden |  |
| UN3293 | HYDRAZINE, AQUEOUS SOLUTION with not more than 37 per cent hydrazine, by mass | 6.1 |  | III |  | 5 |  |  | 60 |  |
| UN3294 | HYDROGEN CYANIDE, SOLUTION IN ALCOHOL with not more than 45 per cent hydrogen cyanide | 6.1 |  | I | 68 | 0 | 1 000 | Forbidden | Forbidden | P |
|  |  | (3) |  |  |  |  |  |  |  |  |
| UN3295 | HYDROCARBONS, LIQUID, N.O.S. | 3 |  | I |  | 0.5 |  | Forbidden | 1 | • |
|  |  |  |  | II |  | 1 |  |  | 5 | • |
|  |  |  |  | III |  | 5 |  |  | 60 | • |
| UN3296 | HEPTAFLUOROPROPANE; or REFRIGERANT GAS R 227 | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3297 | ETHYLENE OXIDE AND CHLOROTETRAFLUOROETHANE MIXTURE with not more than 8.8 per cent ethylene oxide | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3298 | ETHYLENE OXIDE AND PENTAFLUOROETHANE MIXTURE with not more than 7.9 per cent ethylene oxide | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3299 | ETHYLENE OXIDE AND TETRAFLUOROETHANE MIXTURE with not more than 5.6 per cent ethylene oxide | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3300 | ETHYLENE OXIDE AND CARBON DIOXIDE MIXTURE with more than 87 per cent ethylene oxide | 2.3 (2.1) |  |  |  | 0 | 500 | Forbidden | Forbidden |  |
| UN3301 | CORROSIVE LIQUID, SELF-HEATING, N.O.S. | 8 (4.2) |  | I | 16 | 0 | 3 000 | Forbidden | 0.5 | • |
|  |  |  |  | II | 16 | 0 |  | Forbidden | 1 | • |
| UN3302 | 2-DIMETHYLAMINOETHYL ACRYLATE | 6.1 |  | II |  | 0.1 |  | Forbidden | 5 |  |
| UN3303 | COMPRESSED GAS, TOXIC, OXIDIZING, N.O.S. | 2.3 (5.1) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3304 | COMPRESSED GAS, TOXIC, CORROSIVE, N.O.S. | 2.3 (8) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3305 | COMPRESSED GAS, TOXIC, FLAMMABLE, CORROSIVE, N.O.S. | 2.3 (2.1) (8) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3306 | COMPRESSED GAS, TOXIC, OXIDIZING, CORROSIVE, N.O.S. | 2.3 (5.1) (8) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3307 | LIQUEFIED GAS, TOXIC, OXIDIZING, N.O.S. | 2.3 (5.1) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3308 | LIQUEFIED GAS, TOXIC, CORROSIVE, N.O.S. | 2.3 (8) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3309 | LIQUEFIED GAS, TOXIC, FLAMMABLE, CORROSIVE, N.O.S. | 2.3 (2.1) (8) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3310 | LIQUEFIED GAS, TOXIC, OXIDIZING, CORROSIVE, N.O.S. | 2.3 (5.1) (8) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3311 | GAS, REFRIGERATED LIQUID, OXIDIZING, N.O.S. | 2.2 (5.1) |  |  | 16 | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden | • |
| UN3312 | GAS, REFRIGERATED LIQUID, FLAMMABLE, N.O.S. | 2.1 |  |  | 16 | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden | • |
| UN3313 | ORGANIC PIGMENTS, SELF-HEATING | 4.2 |  | II |  | 0 |  |  | 15 |  |
|  |  |  |  | III |  | 0 |  |  | 25 |  |
| UN3314 | PLASTICS MOULDING COMPOUND in dough, sheet or extruded rope form evolving flammable vapour | 9 |  | III |  | 5 |  |  | 100 |  |
| UN3315 | CHEMICAL SAMPLE, TOXIC, liquid or solid | Forbidden |  |  |  |  |  |  |  |  |
| UN3316 | CHEMICAL KIT; or FIRST AID KIT | 9 |  |  | 65 | 0 |  |  | 10 |  |
| UN3317 | 2-AMINO-4,6-DINITROPHENOL, WETTED with not less than 20 per cent water, by mass | 4.1 |  | I | 38, 62 | 0 | 75 | Forbidden | 1 |  |
| UN3318 | AMMONIA SOLUTION, relative density less than 0.880 at 15 °C in water, with more than 50 per cent ammonia | 2.3 (8) |  |  |  | 0 | 3 000 | Forbidden | Forbidden |  |
| UN3319 | NITROGLYCERIN MIXTURE, DESENSITIZED, SOLID, N.O.S. with more than 2 per cent but not more than 10 per cent nitroglycerin, by mass | Forbidden |  |  |  |  |  |  |  |  |
| UN3320 | SODIUM BOROHYDRIDE AND SODIUM HYDROXIDE SOLUTION, with not more than 12 per cent sodium borohydride and not more than 40 per cent sodium hydroxide, by mass | 8 |  | II |  | 1 |  |  | 1 |  |
|  |  |  |  | III |  | 5 |  |  | 5 |  |
| UN3321 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-II), non-fissile or fissile excepted | 7 |  |  | 74 | 0 | 100 SOR/2002-306 |  | Forbidden |  |
| UN3322 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-III), non-fissile or fissile excepted | 7 |  |  | 74 | 0 | 100 SOR/2002-306 |  | Forbidden |  |
| UN3323 | RADIOACTIVE MATERIAL, TYPE C PACKAGE, non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  | Forbidden |  |
| UN3324 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-II), FISSILE | 7 |  |  | 74 | 0 | 25 |  | Forbidden |  |
| UN3325 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY, (LSA-III), FISSILE | 7 |  |  | 74 | 0 | 25 |  | Forbidden |  |
| UN3326 | RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-I), FISSILE; or RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-II), FISSILE | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3327 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, FISSILE, non-special form | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3328 | RADIOACTIVE MATERIAL, TYPE B(U) PACKAGE, FISSILE | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3329 | RADIOACTIVE MATERIAL, TYPE B(M) PACKAGE, FISSILE | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3330 | RADIOACTIVE MATERIAL, TYPE C PACKAGE, FISSILE | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3331 | RADIOACTIVE MATERIAL, TRANSPORTED UNDER SPECIAL ARRANGEMENT, FISSILE | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3332 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, SPECIAL FORM, non-fissile or fissile excepted | 7 |  |  | 74 | 0 |  |  | Forbidden |  |
| UN3333 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, SPECIAL FORM, FISSILE | 7 |  |  | 74 | 0 | 0 |  | Forbidden |  |
| UN3336 | MERCAPTAN MIXTURE, LIQUID, FLAMMABLE, N.O.S.; or MERCAPTANS, LIQUID, FLAMMABLE, N.O.S. | 3 |  | I | 16 | 0 |  | Forbidden | 1 | • |
|  |  |  |  | II | 16 | 1 |  |  | 5 | • |
|  |  |  |  | III | 16 | 5 |  |  | 60 | • |
| UN3337 | REFRIGERANT GAS R 404A | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3338 | REFRIGERANT GAS R 407A | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3339 | REFRIGERANT GAS R 407B | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3340 | REFRIGERANT GAS R 407C | 2.2 |  |  |  | 0.125 SOR/2002-306 |  |  | 75 |  |
| UN3341 | THIOUREA DIOXIDE | 4.2 |  | II |  | 0 |  | Forbidden | 15 |  |
|  |  |  |  | III |  | 0 |  | Forbidden | 25 |  |
| UN3342 | XANTHATES | 4.2 |  | II |  | 0 |  | Forbidden | 15 |  |
|  |  |  |  | III |  | 0 |  | Forbidden | 25 |  |
| UN3343 | NITROGLYCERIN MIXTURE, DESENSITIZED, LIQUID, FLAMMABLE, N.O.S. with not more than 30 per cent nitroglycerin, by mass | Forbidden |  |  |  |  |  |  |  |  |
| UN3344 | PENTAERYTHRITE TETRANITRATE MIXTURE, DESENSITIZED, SOLID, N.O.S. with more than 10 per cent but not more than 20 per cent pentaerythrite tetranitrate (PETN), by mass | Forbidden |  |  |  |  |  |  |  |  |
| UN3345 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 |  |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 |  |
|  |  |  |  | III | 16 | 5 |  |  | 100 |  |
| UN3346 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden |  |
|  |  |  |  | II | 16 | 1 |  |  | 1 |  |
| UN3347 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16 | 0 | 1 000 |  | 1 | P |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 | P |
|  |  |  |  | III | 16 | 1 |  |  | 60 | P |
| UN3348 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 |  |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 |  |
|  |  |  |  | III | 16 | 5 |  |  | 60 |  |
| UN3349 | PYRETHROID PESTICIDE, SOLID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 5 |  |
|  |  |  |  | II | 16 | 0.5 |  |  | 25 |  |
|  |  |  |  | III | 16 | 5 |  |  | 100 |  |
| UN3350 | PYRETHROID PESTICIDE, LIQUID, FLAMMABLE, TOXIC, flash point less than 23 °C | 3 (6.1) |  | I | 16 | 0 | 1 000 |  | Forbidden |  |
|  |  |  |  | II | 16 | 1 |  |  | 1 |  |
| UN3351 | PYRETHROID PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flash point not less than 23 °C | 6.1 (3) |  | I | 16 | 0 | 1 000 |  | 1 |  |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 |  |
|  |  |  |  | III | 16 | 1 |  |  | 60 |  |
| UN3352 | PYRETHROID PESTICIDE, LIQUID, TOXIC | 6.1 |  | I | 16 | 0 | 1 000 |  | 1 |  |
|  |  |  |  | II | 16 | 0.1 |  |  | 5 |  |
|  |  |  |  | III | 16 | 5 |  |  | 60 |  |
| UN3353 | REPEALED [SOR/2002-306] |  |  |  |  |  |  |  |  |  |
| UN3354 | INSECTICIDE GAS, FLAMMABLE, N.O.S. | 2.1 |  |  | 16 | 0.125 SOR/2002-306 | 3 000 | Forbidden | Forbidden | • |
| UN3355 | INSECTICIDE GAS, TOXIC, FLAMMABLE, N.O.S. | 2.3 (2.1) |  |  | 16, 38 | 0 | 0 | Forbidden | Forbidden | • |
| UN3356 | OXYGEN GENERATOR, CHEMICAL | 5.1 |  | II | 41 | 1 |  | Forbidden | Forbidden |  |
| UN3357 | NITROGLYCERIN MIXTURE, DESENSITIZED, LIQUID, N.O.S. with not more than 30 per cent nitroglycerin, by mass | Forbidden |  |  |  |  |  |  |  |  |
| UN3358 | REFRIGERATING MACHINES containing flammable, non-toxic, liquefied gas | 2.1 |  |  |  | 0 |  |  | Forbidden |  |